Programming the DRV8818EVM USB to Serial Port, Serial Number

To program the DRV8818EVM FTDI USB chip, the program MPROG.exe will be used. This software communicates with the FTDI driver and allows for the serial number programming.

The serial number programming is important because each USB to Serial FTDI chip comes programmed with a default and totally different Serial Number. As a result, plugging two or more different boards into the same computer asks Windows to undergo the enumeration and driver installation on a per board basis.

By programming the serial number for all boards, to the same sequence (TICMS0), each board then looks identical to the others, and the driver installation process has to be done only once on a per computer basis.

Steps:

- Unzip the archive MProg3.0_Setup.zip provided in the package.

- Install MPROG 3.0_Setup.exe on your computer.

- Start MPROG 3.0. Screen should look like this:

- On the File menu, go to OPEN

- Select the file DRV88xx_Serial.ept, provided on this package, and press OK. Screen should look like this:

7

- Connect the DRV8818EVMboard to the USB port. Board’s MSP430 microcontroller can be programmed or non programmed at this stage.

- Under Device, press Scan (or press CTRL+C, or press the magnifier lens button).

- MPROG will search for the FTDI chip. It may take a few retries of ITEM 7 before the FTDI chip is found. Once it is, the screen will look like this:

9

10

- Notice that Number of “Blank Devices” or “Number of Programmed” Devices should be equal to 1.

- For the last step, under Device, press Program (or CTRL +P or press the program button).

- Once the device is programmed, the screen will look like this

- DRV8818EVM board Serial Number is programmed and can now be unplugged.

- Repeat steps 6-12 for more DRV8818EVM Boards.

Programming DRV8818EVM MSP430F2617 Firmware

The DRV8818EVM EVM houses an MSP430F2617 microcontroller to control the DRV8818PWP device. This microcontroller needs to be programmed after the EVM is manufactured. To burn the firmware into the microcontroller, the program Lite FETPro430 from ElProtonic can be used.

A TI MSP430 USB Debug Interface must be used. The recommended part number is the MSP-FET430UIF.

- Unzip the archive FET-Pro430-Lite-Setup.zip provided in the package.

- Install Lite FETPro430 by double clicking the extracted Setup.exe file.

- Connect the MSP-FET430UIF to your computer using an USB Cable.

- Start Lite FETPro430. Screen should look like this:

5

- Default Port is LPT1. We must change this to USB Automatic. Go to the Setup Menu and press Connection/Device Reset. The following screen appears.

6

- Select USB Automatic, as shown on the picture above and press OK

- Back on the main screen, locate the Microcontroller Type Frame. Make sure the group is MSP430F2xx and the MSP430F- is set to 2617

7

- Press the Open Code File button which will show you the Open File Dialog. Search for the DRV8818.D43 file provided with this package, select it and press OPEN.

- You may want to save this setup for future programming sessions. To do so, on the File Menu, press Save Setup As. The Save Dialog appears. Enter a meaningful name and a desired location to save the INI file and press SAVE.

The next steps are performed on the Customer EVM hardware:

- Before programming the board, we must place a jumper on the DRV8818EVM Customer EVM’s JMP1 on location 1-2 as seen below.

- Connect the MSP-FET430UIF 14 pin connector to the EVM’s J1 (JTAG Interface) Connector. Make sure the red striped wire on the flat ribbon cable is facing the JTAG connector’s pin 1. Connecting the ribbon cable backwards will result in the lack of communication with the device.

Following steps are performed back on the Lite FETPro430 software:

NOTE: The first time step 12 is executed, it is very possible the FET-430 Windows Application will ask to reprogram the FETUI module’s firmware. If this is the case, please follow the on Screen Instructions to reprogram the module.

- Press the “Auto Prog” Button. The software will erase the FLASH, verify it has been properly erased, burn the firmware into the microcontroller and then verify the firmware was properly downloaded. At the end, the screen should look like this:

13. For programming subsequent DRV8818EVM circuit boards, press F5 or the NEXT button at the bottom right corner.