Customer : 	FortelDTV

Diluth Ga.

Contact:	Jack Curtis

Requirement :

A circuit that allows remote trimming of the gain of a video circuit in steps no larger than 0.5% from 50% to 150%.

The Circuit design provided for this requirement is a complete National solution using the following parts:

DAC081S101 General purpose 8 bit DAC.

LMP2014MT Quad Precision Rail to Rail Output Op Amp.

LMH6714  Wide Band Video Op Amp.

LMH6505 Wide Band VGA.

Theory of Operation:

VGA:

The input voltage to the VGA circuit is set to be 1V p-p and can be adjusted from

0.5V p-p  to 1.5V p-p.

The VGA maximum gain is set to be 10.

The gain control range chosen for  the VGA is from 2 to 6.

On the input to the VGA there is a 75 ohm pad with attenuation of 0.25 (-12dB).

The output buffer has a gain of 2 to compensate for the 75 ohm transmission line and termination.

With  a -12dB pad and Gain from +6dB to +15.5 the overall gain is from -6dB +3.5dB

or .5 to 1.5 per the spec.

Scaling:

The precision op amp is used to scale the output of the DAC to DC control voltages that create gains from 2 to 6 on the VGA.

D to A :

The D to A is an 8 bit general purpose DAC  with a reference voltage of 5Vdc (V+) and a Rail to Rail Output so the LSB value will be 5/256 = 19.5mVDC.

The VGA Gain from 2 to 6 is controlled by the 256 steps of the DAC.

Summary:

This circuit provides more than adequate granularity for gain adjustment.

The circuit requires some calibration of the control voltage to the VGA to accurately set the start and stop points of the VGA.  This can be done with potentiometers or can be automated with the use of additional DACs.

Bandwidth and THD are sufficient for most analog video signals.