LMK03806 Evaluation Board

User's Guide

November 2013

SNAU075A

LMK03806

Ultra-low Jitter Clock Generator with 14 Outputs

Evaluation Board Operating Instructions

contents

1.	Introduction	5

2.	Quick Start	6

3.	Default CodeLoader Modes for Evaluation Boards	7

4.	Example: Using CodeLoader to Program the LMK03806B	8

5.	Program/Load Device	9

6.	Restoring a Default Mode	9

7.	Visual Confirmation of Frequency Lock	10

8.	Enable Clock Outputs	10

9.	PLL Loop Filters and Loop Parameters	11

10.	Evaluation Board Inputs and Outputs	12

12.	CodeLoader Usage	16

13.	Typical Phase Noise Performance Plots	23

14.	Schematics	27

15.	PCB Layers Stackup	35

16.	PCB Layout	36

Appendix A: EVM Software and Communication: Interfacing uWire	41

List of figures

Figure 1: Quick Start Diagram	6

Figure 2: Selecting a Default Mode for the LMK03806B Device	7

Figure 4: Loading the Device	9

Figure 5: Setting the Default mode for LMK03806	9

Figure 6: Setting Digital Delay, Clock Divider, Analog Delay, and Output Format for CLKout0	10

Figure 7: Setting LVCMOS modes	10

Figure 8: Port Setup Tab	16

Figure 9: Clock Outputs Tab	17

Figure 10: PLL Tab	18

Figure 11: Bits/Pins Tab	20

Figure 12: Registers Tab	22

Figure 13: LMK03806B PLL VCO div2 LVPECL Phase Noise	23

Figure 14: LMK03806B div8 CLKout LVPECL Phase Noise	25

Figure 15: LMK03806B div8 CLKout LVDS Phase Noise	25

Figure 16: LMK03806B div8 CLKout LVCMOS Phase Noise	26

Figure 17 - LMK03806 Power Supply Schematic	27

Figure 18 - LMK03806 Device Schematic	28

Figure 19 - Outputs, (OSCout, CLKout0/1/2/3) Schematics	29

Figure 20 - LMK03806 Clock Outputs 4 through 7 Schematics	30

Figure 21 - LMK03806 Clock Outputs 8 through 11 Schematics	31

Figure 22: PCB Stackup	35

Figure 23: Layer 1 - Top	36

Figure 24: Layer 2 – RF Ground Plane	37

Figure 25: Layer 3 – Vcc Planes	38

Figure 26: Layer 4 - Bottom	39

Figure 27: Top and Bottom (Composite)	40

List of Tables

Table 1: EVM Contents	5

Table 2: Default CodeLoader Modes for LMK03806	7

Table 3: PLL Loop Filter Parameters for LMK03806B	11

Table 4: Evaluation Board Inputs and Outputs	12

Table 5: Registers Controls and Descriptions in PLL Tab	18

Table 6: Datasheet to PCB Silkscreen Updates	20

Table 7: Register Controls and Descriptions on Bits/Pins Tab	21

Table 8: LMK03806B PLL VCO div2 Phase Noise and RMS Jitter (fs)	23

Table 9: Typical Phase Noise Performance Plot Setup	24

Table 10: LMK03806B Phase Noise and RMS Jitter for Different CLKout Output Formats and Frequencies	24

Table 11: Bill of Materials for LMK03806BEVAL Boards	32

# Introduction

The Texas Instruments LMK03806BEVAL evaluation module (EVM) helps designers evaluate the operation and performance of the LMK03806B high performance, ultra low-jitter, multirate clock generator. Texas Instruments CodeLoader software can be used to program the internal registers of the LMK03806B device through the USB2ANY-uWire interface.  The CodeLoader software will run on a Windows 7 or Windows XP PC and can be downloaded from http://www.ti.com/tool/codeloader

The EVM contains (See Table 1):

Table 1: EVM Contents

| Quantity | DESCRIPTION |
| --- | --- |
| 1 | LMK03806BEVAL |
| 1 | LKM03806 Quick Start Guide |
| 1 | Interface cables (LPT or USB) |

# Quick Start

  - Connect a voltage of 5.0 volts to the Vcc SMA connector or terminal block.  Device operates at 3.3 V using onboard LP3878-ADJ LDO.

  - Connect the uWire header via LPT or USB2ANY-uWire (See “EVM Software and Communication” Section for more information).

  - Program the device with CodeLoader.  CodeLoader is available for download at: www.ti.com/tool/codeloader

    - Select correct LMK03806B from “Select Device  Clock Conditioners” Menu.

    - Select a default mode from the “Mode” Menu.  For the quick start use, “100 MHz TCXO/XO Reference”

    - Ctrl-L must be pressed at least once to load all registers.  Alternatively click menu Keyboard Controls  Load Device.

  - Measurements may be made at an active CLKout port via its SMA connector. Please see also

.

Figure 1: Quick Start Diagram

# Default CodeLoader Modes for Evaluation Boards

CodeLoader saves the state of the selected LMK03806B device when exiting the software.  To ensure a common starting point, the following modes listed in Table 2: Default CodeLoader Modes for LMK03806 may be restored by clicking “Mode” and selecting the appropriate device configuration, as shown in Figure 2 in the case of the LMK03806B device.  Similar default modes are available for each LMK03806B device in CodeLoader.

Figure 2: Selecting a Default Mode for the LMK03806B Device

After restoring a default mode, press Ctrl+L to program the device.  The default modes also disable certain outputs, so make sure to enable the output under test to make measurements.

Table 2: Default CodeLoader Modes for LMK03806

| Default CodeLoader Mode | XO Frequency |
| --- | --- |
| LMK03806B, 100 MHz | 100 MHz |

The next section outlines step-by-step procedures for using the evaluation board with the LMK03806B.

# Example: Using CodeLoader to Program the LMK03806B

The purpose of this section is to walk the user through using CodeLoader 4 to make some measurements with the LMK03806B device as an example.  For more information on CodeLoader refer to CodeLoader Usage or the CodeLoader 4 instructions located at http://www.ti.com/tool/codeloader.

Before proceeding, be sure to follow the Quick Start section to ensure proper connections.

## 1. Start CodeLoader 4 Application

Click “Start”  “Programs”  “CodeLoader 4”  “CodeLoader 4”

The CodeLoader 4 program is installed by default to the CodeLoader 4 application group.

## 2. Select Device

Click “Select Device”  “Clock Conditioners”  “LMK03806B”

Once started CodeLoader 4 will load the last used device.  To load a new device click “Select Device” from the menu bar, then select the subgroup and finally device to load.  For this example, the LMK03806B is chosen.  Selecting the device does cause the device to be programmed.

# Program/Load Device

Assuming the Port Setup settings are correct, press the “Ctrl+L” shortcut or click “Keyboard Controls”  “Load Device” from the menu to program the device to the current state of the newly loaded LMK03806 file.

Figure 4: Loading the Device

Once the device has been initially loaded, CodeLoader will automatically program changed registers so it is not necessary to re-load the device upon subsequent changes in the device configuration.  It is possible to disable this functionality by ensuring there is no checkmark by the “Options”  “AutoReload with Changes.”

Because a default mode will be restored in the next step, this step isn’t really needed but included to emphasize the importance of pressing “Ctrl+L” to load the device at least once after starting CodeLoader, restoring a mode, or restoring a saved setup using the File menu.

See CodeLoader Usage or the CodeLoader 4 instructions located at http://www.ti.com/tool/codeloader  for more information on Port Setup.  Troubleshooting Information contains information on troubleshooting communications.

# Restoring a Default Mode

Click “Mode”  “100 MHz XO/TCXO Reference”; then press Ctrl+L.

Figure 5: Setting the Default mode for LMK03806

For the purpose of this walkthrough, a default mode will be loaded to ensure a common starting point.  This is important because when CodeLoader is closed, it remembers the last settings used for a particular device.  Again, remember to press Ctrl+L as the first step after loading a default mode.

# Visual Confirmation of Frequency Lock

After a default mode is restored and loaded, LED D4, should illuminate red when the PLL is locked to the reference crystal.

# Enable Clock Outputs

While the LMK03806B offers programmable clock output buffer formats, the evaluation board is shipped with preconfigured output terminations to match the default buffer type for each output.  Refer to the CLKout port description in the Evaluation Board Inputs and Outputs section.

To measure phase noise at one of the clock outputs, for example, CLKout0:

- Click on the Clock Outputs tab,

- Uncheck “Powerdown” in the Divider Powerdown box to enable the channel,

- Set the following settings as needed:

  - Clock Divider value

  - Clock Output type.

Figure 6: Setting Digital Delay, Clock Divider, Analog Delay, and Output Format for CLKout0

- Depending on the configured output type, the clock output SMAs can be interfaced to a test instrument with a single-ended 50-ohm input as follows.

  - For LVDS:

    - A balun (like ADT2-1T) is recommended for differential-to-single-ended conversion.

  - For LVPECL:

    - A balun can be used, or

    - One side of the LVPECL signal can be terminated with a 50-ohm load and the other side can be run single-ended to the instrument.

  - For LVCMOS:

    - There are two single-ended outputs, CLKoutX and CLKoutX*, and each output can be set to Normal, Inverted, or Off.  There are nine (9) combinations of LVCMOS modes in the Clock Output list.

Figure 7: Setting LVCMOS modes

One side of the LVCMOS signal can be terminated with a 50-ohm load and the other side can be run single-ended to the instrument.

    - A balun may also be used.  Ensure CLKoutX and CLKoutX* states are complementary, i.e.: Norm/Inv or Inv/Norm.

- The phase noise may be measured with a spectrum analyzer or signal source analyzer.

See Typical Phase Noise Performance Plots for phase noise plots of the clock outputs.  TI’s Clock Design Tool can be used to calculate divider values to achieve desired clock output frequencies.  See: http://www.ti.com/tool/codeloader.

# PLL Loop Filters and Loop Parameters

The default loop filter for the PLL has been configured for a 60 kHz bandwidth. The following table contains the parameters for the PLL.

TI’s Clock Design Tool can be used to optimize PLL phase noise/jitter for given specifications.  See: http://www.ti.com/tool/codeloader.

## PLL Loop Filter

Table 3: PLL Loop Filter Parameters for LMK03806B

|  | ### Integrated VCO PLL |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | 20 MHz Reference | 100 MHz Reference |  |
| C1_LF |  | 0.022 | .022 | nF |
| C2_LF |  | 18 | 18 | nF |
| C3 (internal) |  | 0.01 | 0.01 | nF |
| C4 (internal) |  | 0.01 | 0.01 | nF |
| R2_A2 |  | 0.82 | 0.82 | kΩ |
| R3 (internal) |  | 0.2 | 0.2 | kΩ |
| R4 (internal) |  | 0.2 | 0.2 | kΩ |
| Charge Pump Current, K |  | 3.2 | 3.2 | mA |
| Phase Detector Frequency |  | 20 | 100 | MHz |
| Frequency |  | 2500 | 2400 | MHz |
| Kvco |  | 19 | 19 | MHz/V |
| N |  | 25 | 12 |  |
| P |  | 5 | 2 |  |
| Phase Margin |  | 75 | 70 | degrees |
| Loop Bandwidth |  | 63 | 60 | kHz |

Note: PLL Loop Bandwidth is a function of K, Kvco, N as well as loop components. Changing K and N will change the loop bandwidth.

# Evaluation Board Inputs and Outputs

The following table contains descriptions of the inputs and outputs for the evaluation board.  Unless otherwise noted, the connectors described can be assumed to be populated by default.  Additionally, some applicable CodeLoader programming controls are noted for convenience.

Table 4: Evaluation Board Inputs and Outputs

| Connector Name | Signal Type, Input/Output | Description |
| --- | --- | --- |
| SMAs Populated: CLKout0, CLKout0*, CLKout2, CLKout2*, CLKout4, CLKout4*, CLKout6, CLKout6*, CLKout8, CLKout8*, CLKout10, CLKout10* SMAs Not Populated: CLKout1, CLKout1*, CLKout3, CLKout3*, CLKout5, CLKout5*, CLKout7, CLKout7*, CLKout9, CLKout9*, CLKout11, CLKout11* | Analog, Output | Clock outputs with programmable output buffers. The output terminations by default on the evaluation board are shown below, and the output type selected by default in CodeLoader is indicated by an asterisk (*): Clock output pair; Default Board Termination. CLKout0; LVPECL*. CLKout1; LVPECL. CLKout2; LVPECL*. CLKout3; LVPECL. CLKout4; LVDS* / LVCMOS. CLKout5; LVDS / LVCMOS. CLKout6; LVDS* / LVCMOS. CLKout7; LVDS / LVCMOS. CLKout8; LVDS* / LVCMOS. CLKout9; LVDS / LVCMOS. CLKout10; LVPECL*. CLKout11; LVPECL. Each CLKout pair has a programmable LVDS, LVPECL, or LVCMOS buffer. The output buffer type can be selected in CodeLoader in the Clock Outputs tab via the CLKoutX_TYPE control. All clock outputs are AC-coupled to allow safe testing with RF test equipment. All LVPECL clock outputs are source-terminated using 240-ohm resistors. If an output pair is programmed to LVCMOS, each output can be independently configured (normal, inverted, or off/tri-state). |
| OSCout0, OSCout0*, OSCout1, OSCout1* | Analog, Output | Buffered outputs of OSCin port. The output terminations on the evaluation board are shown below, the output type selected by default in CodeLoader is indicated by an asterisk (*): OSC output pair; Default Board Termination. OSCout0; LVPECL* (fixed). OSCout1; LVPECL* (fixed). Only OSCout0 has a programmable LVDS, LVPECL, or LVCMOS output buffer. The OSCout0 buffer type can be selected in CodeLoader on the Clock Outputs tab via the OSCout0_TYPE control. OSCout1 has LVPECL buffer only but has programmable swing amplitude. Both OSCout pairs are AC-coupled to allow safe testing with RF test equipment. The OSCout0 and OSCout1 outputs are source-terminated using 240-ohm resistors. If OSCout0 is programmed as LVCMOS, each output can be independently configured (normal, inverted, inverted, and off/tri-state). |
| Vcc | Power, Input | Main power supply input for the evaluation board. A 3.9 V DC power source applied to this SMA will, by default, source the onboard LDO regulators that power the inner layer planes that supply the LMK03806B. The LMK03806B contains internal voltage regulators for the VCO, PLL and other internal blocks. The clock outputs do not have an internal regulator, so a clean power supply with sufficient output current capability is required for optimal performance. On-board LDO regulators and 0  resistor options provide flexibility to supply and route power to various devices. See schematics for more details. |
| J1 | Power, Input | Alternative power supply input for the evaluation board using two unshielded wires (Vcc and GND). Apply power to either Vcc SMA or J1, but not both. |
| OSCin, OSCin* | Analog, Input | By default, these SMAs are not connected to the traces going to the OSCin/OSCin* pins of the LMK03806B. Instead, the onboard crystal drives the OSCin input of the device. A single-ended or differential signal may be used to drive the OSCin/OSCin* pins and must be AC coupled. If operated in single-ended mode, the unused input must be connected to GND with 0.1 uF. Refer to the LMK03806 Datasheet section “Electrical Characteristics” for PLL Reference Input (OSCin) specifications. |
| uWire | CMOS, Input/Output | 10-pin header for uWire programming interface and programmable logic I/O pins for the LMK03806B. The uWire interface includes CLKuWire, DATAuWire, and LEuWire signals. The programmable logic I/O signals accessible through this header include: SYNC. SYNC also has a dedicated SMA and test point. |
| SYNC | CMOS, Input/Output | Programmable status I/O pin. By default, set as an input pin for synchronize the clock outputs with a fixed and known phase relationship between each clock output selected for SYNC. In the default CodeLoader mode, SYNC will asserted when the SYNC pin is low and the outputs to be synchronized will be held in a logic low state. When SYNC is unasserted, the clock outputs to be synchronized are activated and will be initially phase aligned with each other except for outputs programmed with different digital delay values. A SYNC event can also be programmed by toggling the SYNC_POL_INV bit in the Bits/Pins tab in CodeLoader. Refer to the LMK03806 Datasheet section “Clock Output Synchronization” for more information. |

## Recommended Test Equipment

Power Supply

The Power Supply should be a low noise power supply, particularly when the devices on the board are being directly powered (onboard LDO regulators bypassed).

Phase Noise / Spectrum Analyzer

To measure phase noise and RMS jitter, an Agilent E5052 Signal Source Analyzer is recommended.  An Agilent E4445A PSA Spectrum Analyzer with the Phase Noise option is also usable although the architecture of the E5052 is superior for phase noise measurements.  At frequencies less than 100 MHz the local oscillator noise of the E4445A is too high and measurements will reflect the E4445A’s internal local oscillator performance, not the device under test.

Oscilloscope

To measure the output clocks for AC performance, such as rise time or fall time, propagation delay, or skew, it is suggested to use a real-time oscilloscope with at least 1 GHz analog input bandwidth (2.5+ GHz recommended) with 50 ohm inputs and 10+ Gsps sample rate.  To evaluate clock synchronization or phase alignment between multiple clock outputs, it’s recommended to use phase-matched, 50-ohm cables to minimize external sources of skew or other errors/distortion that may be introduced if using oscilloscope probes.

# CodeLoader Usage

Code Loader is used to program the evaluation board with either an LPT or USB2ANY-uWire interface available from http://www.ti.com.

## Port Setup Tab

Figure 8: Port Setup Tab

On the Port Setup tab, the user may select the type of communication port (LPT or USB) that will be used to program the device on the evaluation board.

The Pin Configuration field is hardware dependent and normally does not need to be changed by the user.  Figure 8: Port Setup Tab shows the default settings.

## Clock Outputs Tab

Figure 9: Clock Outputs Tab

The Clock Outputs tab allows the user to control the output channel blocks, including:

- Clock Group Source from either Crystal or OSCin

- Channel Powerdown (affects clock divider, and buffer blocks)

- Clock Divide value

- Clock Output format (per output)

Clicking on the cyan-colored PLL block that contains R, PDF and N values will bring the PLL tab into focus where these values may be modified, if needed.

Clicking on the values in the box containing the Internal Loop Filter component (R3, C3, R4, C4) allow one to step through the possible values.  Left click to increase the component value, and right click to decrease the value.  These values can also be changed in the Bits/Pins tab.

The Reference Oscillator value field may be changed in either the Clock Outputs tab or the PLL tab.  The PLL Reference frequency should match the frequency of the onboard Crystal.

## PLL Tab

Figure 10: PLL Tab

The PLL tab allows the user to change the following parameters in Table 5.

Table 5: Registers Controls and Descriptions in PLL Tab

| Control Name | Register Name | Description |
| --- | --- | --- |
| Reference Oscillator Frequency (MHz) | OSCin_FREQ | OSCin frequency from the External OSCin connector or Crystal. |
| Phase Detector Frequency (MHz) | n/s | PLL Phase Detector Frequency (PDF). This value is calculated as: PLL PDF = OSCin Frequency *(2EN_PLL_REF_2X) / (PLL_R). |
| VCO Frequency (MHz) | n/a | Internal VCO Frequency should be within the allowable range of the LMK03806B device. This value is calculated as: VCO Frequency = PLL PDF * (PLL_N * PLL_P). |
| Doubler | EN_PLL_REF_2X | PLL Doubler. 0 = Bypass Doubler 1 = Enable Doubler |
| R Counter | PLL_R | PLL R Counter value (1 to 4095). |
| N Counter | PLL_N | PLL N Counter value (1 to 49140). |
| OSCout Divider | PLL_P | PLL N Prescaler value (2 to 8). |
| Phase Detector Polarity | PLL_CP_POL | PLL Phase Detector Polarity. Click on the polarity sign to toggle polarity “+” or “–”. |
| Charge Pump Gain | PLL_CP_GAIN | PLL Charge Pump Gain. Left-click/right-click to increase/decrease charge pump gain (100, 400, 1600, 3200 uA). |
| Charge Pump State | PLL_CP_TRI | PLL Charge Pump State. Click to toggle between Active and Tri-State. |

Changes made on this tab will be reflected in the Clock Outputs tab.  The VCO Frequency should conform to the specified internal VCO frequency range for the LMK03806B.

## Bits/Pins Tab

Figure 11: Bits/Pins Tab

The Bits/Pins tab allows the user to program bits directly, many of which are not available on other tabs.  Brief descriptions for the controls on this tab are provided in Table 7: Register Controls and Descriptions on Bits/Pins Tab to supplement the datasheet.  Refer to the LMK03806 Datasheet for more information.

TIP: Right-clicking any register name in the Bits/Pins tab will display a Help prompt with the register address, data bit location/length, and a brief register description.

Note: Table 6 shows some differences between the datasheet names and PCB names for -002 PCB’s:

Table 6: Datasheet to PCB Silkscreen Updates

| Datasheet Name | PCB Silkscreen Identifier |
| --- | --- |
| Readback (pin 27) | Status0 |
| Ftest/LD (pin 33) | Status1 |
| GPout0 (pin 62) | Status2 |
| GPout1 (pin 63) | Status3 |

Table 7: Register Controls and Descriptions on Bits/Pins Tab

| Group | Register Name | Description |
| --- | --- | --- |
| Mode Control | RESET | Resets the device to default register values. RESET must be cleared for normal operation to prevent an unintended reset every time R0 is programmed. |
|  | POWERDOWN | Places the device in powerdown mode. |
|  | uWire_LOCK | When checked, no other uWire programming will have effect. Must be unchecked to enable uWire programming of registers R0 to R30. |
| Automatic Update | OSCin_FREQ | Sets the OSCin frequency range. |
|  | PLL_N_CAL | Sets the PLL_N value. |
| Crystal | EN_PLL_XTAL | Enables Crystal Oscillator. |
| Output Control | READBACK_TYPE | Readback pin type. (Labeled Stats0 on PCB) |
|  | LD_MUX | Ftest/LD pin selection when output. (Ftest/LD output labeled Status1 on PCB) |
|  | LD_TYPE | Sets I/O pin type on the LD pin. |
|  | GPO0 | Sets logic level on the GPO0 pin. (Labeled Status2 on PCB) |
|  | GPO1 | Sets logic level on the GPO1 pin. (Labeled Status3 on PCB) |
| IO Control – Sync | SYNC_TYPE | Sets I/O pin type on the SYNC pin. |
|  | SYNC_POL_INV | Sets polarity on SYNC input to active low when checked. Toggling this bit will initiate a SYNC event. |
|  | SYNC_PLL_DLD | Engage SYNC mode until PLL DLD is true |
|  | NO_SYNC_CLKoutX_Y | Synchronization will not affect selected clock outputs, where X = even-numbered output and Y = odd-numbered output. |
| PLL | PLL_DLD_CNT | The reference and feedback of PLL must be within the window of phase error as specified by PLL_WND_SIZE for this many cycles before PLL digital lock detect is asserted. |
|  | EN_PLL_REF_2X | Enables the doubler block to doubles the reference frequency into the PLL R counter. This can allow for frequency of 2/3, 2/5, etc. of OSCin to be used at the phase detector of PLL. |
|  | PLL_R3_LF | Set the corresponding integrated PLL loop filter values: R3, R4, C3, and C4. It is also possible to set these values by clicking on the loop filter values on the Clock Outputs tab. |
|  | PLL_R4_LF |  |
|  | PLL_C3_LF |  |
|  | PLL_C4_LF |  |
| Program Pins | SYNC | Sets these pins on the uWire header to logic high (checked) or logic low (unchecked). |
|  | TRIGGER |  |

## Registers Tab

Figure 12: Registers Tab

The Registers tab shows the value of each register.  This is convenient for programming the device to the desired settings, then exporting to a text file the register values in hexadecimal for use in your own application.

By clicking in the “bit field” it is possible to manually change the value of registers by typing ‘1’ and ‘0.’

# Typical Phase Noise Performance Plots

## PLL

Figure 13: LMK03806B PLL VCO div2 LVPECL Phase Noise

Table 8: LMK03806B PLL VCO div2 Phase Noise and RMS Jitter (fs)

| Offset | Phase Noise (dBc/Hz) |
| --- | --- |
| 100 Hz | -98.3 |
| 1 kHz | -107.8 |
| 10 kHz | -106.6 |
| 100 kHz | -114.2 |
| 1 MHz | -136.6 |
| 10 MHz | -150.6 |
| 20 MHz | -151.3 |
| RMS Jitter (fs) 12 kHz to 20 MHz | 215 |
| RMS Jitter (fs) 100 Hz to 20 MHz | 229 |

## Clock Outputs (CLKout)

The LMK03806 Family features programmable LVDS, LVPECL, and LVCMOS buffer modes for the CLKoutX and OSCout0 output pairs.  The OSCout1 output pair has a LVPECL buffer.  Included below are various phase noise measurements for each output format.

### CLKout Phase Noise (div8 and div16)

For the LMK03806B, the internal VCO frequency is 2400 MHz.  The divide-by-8 CLKout frequency is 312.5 MHz, and the divide-by-16 CLKout frequency is 156.25 MHz.

Table 9: Typical Phase Noise Performance Plot Setup

| Parameter | Condition |
| --- | --- |
| LMK03806B Mode | 100 MHz TCXO/XO Reference |
| Loop Filter Parameters | As shown under “100 MHz Reference” in Table 3 |
| CLKout for LVDS/LVCMOS | CLKout8, with CLKout8* terminated in to 50 Ω |
| CLKout for LVPECL | CLKout10, with CLKout10* terminated in to 50 Ω |

Table 10: LMK03806B Phase Noise and RMS Jitter for Different CLKout Output Formats and Frequencies

| Offset | div8 LVPECL | div8 LVDS | div8 LVCMOS | div16 LVPECL | div16 LVDS | div16 LVCMOS |
| --- | --- | --- | --- | --- | --- | --- |
| 100 Hz | -91.9 | -92.0 | -93.2 | -98.6 | -98.8 | -97.1 |
| 1 kHz | -113.8 | -113.2 | -113.4 | -119.8 | -119.3 | -119.0 |
| 10 kHz | -122.6 | -122.7 | -122.5 | -128.7 | -128.4 | -128.4 |
| 100 kHz | -128.7 | -128.9 | -128.4 | -134.8 | -134.9 | -134.4 |
| 1 MHz | -148.1 | -147.7 | -148.2 | -153.7 | -153.0 | -153.7 |
| 10 MHz | -157.6 | -155.0 | -157.2 | -160.5 | -158.0 | -160.4 |
| 20 MHz | -157.7 | -155.1 | -157.2 | -160.7 | -158.1 | -160.4 |
| RMS Jitter (fs) 12 kHz to 20 MHz | 141.1 | 144.0 | 143.2 | 145.3 | 155.4 | 149.8 |
| RMS Jitter (fs) 100 Hz to 20 MHz | 206.1 | 210.5 | 210.2 | 208.8 | 217.1 | 224.4 |

Figure 14: LMK03806B div8 CLKout LVPECL Phase Noise

Figure 15: LMK03806B div8 CLKout LVDS Phase Noise

Figure 16: LMK03806B div8 CLKout LVCMOS Phase Noise

# Schematics

## Power Supplies

Figure 17 - LMK03806 Power Supply Schematic

## LMK03806B Device with Loop Filter and Crystal Circuits

Figure 18 - LMK03806 Device Schematic

## Outputs, (OSCout0/1, CLKout0/1/2/3)

Figure 19 - Outputs, (OSCout, CLKout0/1/2/3) Schematics

## Clock Outputs (CLKout 4/5/6/7)

Figure 20 - LMK03806 Clock Outputs 4 through 7 Schematics

## Clock Outputs (CLKout8/9/10/11)

Figure 21 - LMK03806 Clock Outputs 8 through 11 Schematics

Bill of Materials

Table 11: Bill of Materials for LMK03806BEVAL Boards

| Item | Description | Qty | Designator | Manufacturer | PartNumber |
| --- | --- | --- | --- | --- | --- |
| 1 | CAP, CERM, 47pF, 50V, +/-5%, C0G/NP0, 0603 | 1 | C1_LF | Kemet | C0603C470J5GACTU |
| 2 | CAP, CERM, 3900pF, 50V, +/-10%, X7R, 0603 | 1 | C2_LF | MuRata | GRM188R71H392KA01D |
| 3 | CAP, CERM, 0.1uF, 25V, +/-5%, X7R, 0603 | 33 | C5, C8, C12, C15, C17, C18, C19, C20, C21, C22, C23, C24, C25, C26, C27, C28, C29, C30, C32, C33, C34, C35, C36, C37, C38, C39, C40, C41, C42, C45, C48, C60, C61 | Kemet | C0603C104J3RACTU |
| 4 | CAP, CERM, 22pF, 50V, +/-5%, C0G/NP0, 0603 | 1 | C6 | AVX | 06035A220JAT2A |
| 5 | CAP, CERM, 10uF, 10V, +/-10%, X5R, 0805 | 4 | C7, C43, C46, C56 | Kemet | C0805C106K8PACTU |
| 6 | RES, 0 ohm, 5%, 0.1W, 0603 | 10 | C11, C14, R18, R84, R86, R88, R90, R95, R100, R104 | Vishay-Dale | CRCW06030000Z0EA |
| 7 | RES, 240 ohm, 5%, 0.1W, 0603 | 16 | C13, C16, R27, R28, R34, R35, R36, R37, R38, R39, R44, R45, R75, R76, R77, R78 | Vishay-Dale | CRCW0603240RJNEA |
| 8 | CAP, CERM, 0.1uF, 25V, +/-10%, X7R, 0603 | 1 | C31 | Kemet | C0603C104K3RACTU |
| 9 | CAP, CERM, 1uF, 10V, +/-10%, X5R, 0603 | 3 | C44, C47, C55 | Kemet | C0603C105K8PACTU |
| 10 | CAP, CERM, 4.7uF, 10V, +/-10%, X5R, 0603 | 1 | C53 | Kemet | C0603C475K8PACTU |
| 11 | CAP, CERM, 2200pF, 50V, +/-10%, X7R, 0603 | 1 | C54 | Kemet | C0603C222K5RACTU |
| 12 | CAP, CERM, 0.1uF, 16V, +/-10%, X7R, 0603 | 1 | C58 | Kemet | C0603C104K4RACTU |
| 13 | Connector, SMT, End launch SMA 50 Ohm | 16 | CLKout0, CLKout0*, CLKout2, CLKout2*, CLKout4, CLKout4*, CLKout6, CLKout6*, CLKout8, CLKout8*, CLKout10, CLKout10*, OSCout0, OSCout0*, OSCout1, OSCout1* | Emerson Network Power | 142-0701-851 |
| 14 | LED 2.8X3.2MM 565NM RED CLR SMD | 3 | D1, D2, D4 | Lumex Opto/Components Inc. | SML-LX2832IC |
| 15 | LED 2.8X3.2MM 565NM GRN CLR SMD | 1 | D3 | Lumex Opto/Components Inc. | SML-LX2832GC |
| 16 | CONN TERM BLK PCB 5.08MM 2POS OR | 1 | J1 | Weidmuller | 1594540000 |
| 17 | RES, 620 ohm, 5%, 0.1W, 0603 | 1 | R2_LF | Vishay-Dale | CRCW0603620RJNEA |
| 18 | RES, 15k ohm, 5%, 0.1W, 0603 | 4 | R3, R5, R7, R9 | Vishay-Dale | CRCW060315K0JNEA |
| 19 | RES, 27k ohm, 5%, 0.1W, 0603 | 4 | R4, R6, R8, R10 | Vishay-Dale | CRCW060327K0JNEA |
| 20 | RES, 18 ohm, 5%, 0.1W, 0603 | 2 | R12, R23 | Vishay-Dale | CRCW060318R0JNEA |
| 21 | RES, 270 ohm, 5%, 0.1W, 0603 | 6 | R14, R15, R19, R20, R21, R24 | Vishay-Dale | CRCW0603270RJNEA |
| 22 | RES, 51 ohm, 5%, 0.1W, 0603 | 12 | R33, R42, R43, R46, R48, R54, R56, R64, R66, R72, R74, R80 | Vishay-Dale | CRCW060351R0JNEA |
| 23 | RES, 33 ohm, 5%, 0.1W, 0603 | 2 | R57, R60 | Vishay-Dale | CRCW060333R0JNEA |
| 24 | FB, 120 ohm, 500 mA, 0603 | 9 | R81, R85, R87, R89, R91, R93, R96, R98, R101 | Murata | BLM18AG121SN1D |
| 25 | RES, 0 ohm, 5%, 0.125W, 0805 | 1 | R92 | Vishay-Dale | CRCW08050000Z0EA |
| 26 | RES, 51k ohm, 5%, 0.1W, 0603 | 1 | R94 | Vishay-Dale | CRCW060351K0JNEA |
| 27 | RES, 2.00k ohm, 1%, 0.1W, 0603 | 1 | R97 | Vishay-Dale | CRCW06032K00FKEA |
| 28 | RES, 866 ohm, 1%, 0.1W, 0603 | 1 | R99 | Vishay-Dale | CRCW0603866RFKEA |
| 29 | RES, 1.00k ohm, 1%, 0.1W, 0603 | 2 | R103 | Vishay-Dale | CRCW06031K00FKEA |
| 30 | 0.875" Standoff | 6 | S1, S2, S3, S4, S5, S6 | VOLTREX | SPCS-14 |
| 31 | LMK03806 | 1 | U1 | Texas Instruments | LMK03806BISQ |
| 32 | Micropower 800mA Low Noise 'Ceramic Stable' Adjustable Voltage Regulator for 1V to 5V Applications | 1 | U2 | Texas Instruments | LP3878SD-ADJ |
| 33 | Low Profile Vertical Header 2x5 0.100" | 1 | uWire | FCI | 52601-G10-8LF |
| 34 | Connector, TH, SMA | 1 | Vcc | Emerson Network Power | 142-0701-201 |
| 35 | 100 MHz TCXO | 1 | Y3 | Connor Winfield | CWX813-100.00M |
| 36 | CAP, CERM, 100pF, 50V, +/-5%, C0G/NP0, 0603 | 0 | C1, C2, C3, C4 | Kemet | C0603C101J5GACTU |
| 37 | CAP, CERM, 220pF, 50V, +/-5%, C0G/NP0, 0603 | 0 | C2pLF | MuRata | GRM1885C1H221JA01D |
| 38 | CAP, CERM, 22pF, 50V, +/-5%, C0G/NP0, 0603 | 0 | C9 | AVX | 06035A220JAT2A |
| 39 | CAP, CERM, 0.1uF, 25V, +/-5%, X7R, 0603 | 0 | C10, C62, C64 | Kemet | C0603C104J3RACTU |
| 40 | CAP, CERM, 470pF, 50V, +/-10%, X7R, 0603 | 0 | C49, C51, C57 | Kemet | C0603C471K5RACTU |
| 41 | CAP, CERM, 0.1uF, 25V, +/-10%, X7R, 0603 | 0 | C50, C52, C59 | Kemet | C0603C104K3RACTU |
| 42 | CAP, CERM, 1uF, 25V, +/-10%, X5R, 0805 | 0 | C63, C65 | AVX | 08053D105KAT2A |
| 43 | Connector, SMT, End launch SMA 50 Ohm | 0 | CLKout1, CLKout1*, CLKout3, CLKout3*, CLKout5, CLKout5*, CLKout7, CLKout7*, CLKout9, CLKout9*, CLKout11, CLKout11*, IC_SYNC, OSCin, OSCin* | Emerson Network Power | 142-0701-851 |
| 44 | RES, 270 ohm, 5%, 0.1W, 0603 | 0 | R1, R2 | Vishay-Dale | CRCW0603270RJNEA |
| 45 | RES, 0 ohm, 5%, 0.1W, 0603 | 0 | R11, R16, R22 | Vishay-Dale | CRCW06030000Z0EA |
| 46 | RES, 100 ohm, 5%, 0.1W, 0603 | 0 | R17 | Vishay-Dale | CRCW0603100RJNEA |
| 47 | RES, 51 ohm, 5%, 0.1W, 0603 | 0 | R25, R26, R29, R30, R31, R32, R40, R41, R47, R53, R55, R63, R65, R71, R73, R79, R300 | Vishay-Dale | CRCW060351R0JNEA |
| 48 | RES, 240 ohm, 5%, 0.1W, 0603 | 0 | R49, R50, R51, R52, R58, R59, R61, R62, R67, R68, R69, R70 | Vishay-Dale | CRCW0603240RJNEA |
| 49 | RES, 1.00k ohm, 1%, 0.1W, 0603 | 0 | R102 | Vishay-Dale | CRCW06031K00FKEA |
| 50 | FB, 1000 ohm, 600 mA, 0603 | 0 | R82, R83 | Murata | BLM18HE102SN1D |
| 51 |  | 0 | Y1, Y2 | ECS | DNP_XTAL, ECS-200-20-30B-DU |

# PCB Layers Stackup

6-layer PCB Stackup includes:

- Top Layer for high-priority high-frequency signals (2 oz.)

- FR4 Dielectric, 19 mils

- RF Ground plane (1 oz.)

- FR4, 14.5 mils

- Power plane (1 oz.)

- FR4, 19 mils

- Bottom Layer copper clad for thermal relief (2 oz.)

Figure 22: PCB Stackup

FR4 (Er = 4.8)

19 mil

Top Layer [LMK03806ENG.GTL]

RF Ground plane [LMK03806ENG.G1]

FR4 (Er = 4.8)

14.5 mil

Power plane #1 [LMK03806ENG.G2]

FR4

19 mil

Bottom Layer [LMK03806ENG.GBL]

61.3 mil thick

# PCB Layout

Figure 23: Layer 1 - Top

Figure 24: Layer 2 – RF Ground Plane

Figure 25: Layer 3 – Vcc Planes

Figure 26: Layer 4 - Bottom

Figure 27: Top and Bottom (Composite)

# Appendix A:

# EVM Software and Communication: Interfacing uWire

Codeloader is the software used to communicate with the EVM (Please download the latest version from TI.com - http://www.ti.com/tool/codeloader). This EVM can be controlled through the uWire interface on board. There are two options in communicating with the uWire interface from the computer.

OPTION 1

Open Codeloader.exe  Click “Select Device”  Click “Port Setup” tab  Click “LPT” (in Communication Mode)

OPTION 2

The Adapter Board

This table describes the pins configuration on the adapter board for each EVM board (See examples below table)

| EVM | Jumper Bank |  |  |  |  |  |  |  | Code Loader Configuration |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A | B | C | D | E | F | G | H |  |
| LMX2581 | A4 | B1 | C2 |  | E5 | F1 | G1 | H1 | BUFEN (pin 1), Trigger (pin 7) |
| LMX2541 | A4 |  | C3 |  | E4 | F1 | G1 | H1 | CE (pin 1), Trigger (pin 10) |
| LMK0400x | A0 |  | C3 |  | E5 | F1 | G1 | H1 | GOE (pin 7) |
| LMK01000 | A0 |  | C1 |  | E5 | F1 | G1 | H1 | GOE (pin 7) |
| LMK030xx | A0 |  | C1 |  | E5 | F1 | G1 | H1 | SYNC (pin 7) |
| LMK02000 | A0 |  | C1 |  | E5 | F1 | G1 | H1 | SYNC (pin 7) |
| LMK0480x | A0 | B2 | C3 |  | E5 | F0 | G0 | H1 | Status_CLKin1 (pin 3) |
| LMK04816/4906 | A0 | B2 | C3 |  | E5 | F0 | G0 | H1 | Status_CLKin1 (pin 3) |
| LMK01801 | A0 | B4 | C5 |  | E2 | F0 | G0 | H1 | Test (pin 3), SYNC0 (pin 10) |
| LMK0482x (prelease) | A0 | B5 | C3 | D2 | E4 | F0 | G0 | H1 | CLKin1_SEL (pin 6), Reset (pin 10) |
| LMX2531 | A0 |  |  |  | E5 | F2 | G1 | H2 | Trigger (pin 1) |
| LMX2485/7 | A0 |  | C1 |  | E5 | F2 | G1 | H0 | ENOSC (pin 7), CE (pin 10) |
| LMK03200 | A0 |  |  |  | E5 | F0 | G0 | H1 | SYNC (pin 7) |
| LMK03806 | A0 |  | C1 |  | E5 | F0 | G0 | H1 |  |
| LMK04100 | A0 |  | C1 |  | E5 | F1 | G1 | H1 |  |

Example adapter configuration (LMK01801)

Open Codeloader.exe  Click “Select Device”  Click “Port Setup” Tab  Click “USB” (in Communication Mode)

*Remember to also make modifications in “Pin Configuration” Section according to Table above.