LMK048xx Evaluation Board

User's Guide

November 2013

SNAU158

Low-Noise Clock Jitter Cleaner with Dual Loop PLLs

Evaluation Board Operating Instructions

# Table of Contents

Table of Contents	3

General Description	6

Evaluation Board Kit Contents	6

Available LMK048xx Evaluation Boards	6

Available LMK04800 Family Devices	6

Quick Start	7

Default CodeLoader Modes for Evaluation Boards	8

Example: Using CodeLoader to Program the LMK04808B	9

1. Start CodeLoader 4 Application	9

2. Select Device	9

3. Program/Load Device	9

4. Restoring a Default Mode	10

5. Visual Confirmation of Frequency Lock	11

6. Enable Clock Outputs	11

PLL Loop Filters and Loop Parameters	13

PLL 1 Loop Filter	13

122.88 MHz VCXO PLL	13

PLL2 Loop Filter	14

Integrated VCO PLL	14

Evaluation Board Inputs and Outputs	15

Recommended Test Equipment	23

Programming 0-Delay Mode in CodeLoader	24

Overview	24

Dual Loop 0-Delay Mode Examples	24

Programming Steps	24

Details	24

Single Loop 0-Delay Mode Examples	26

Programming Steps	26

Details	26

Appendix A: CodeLoader Usage	28

Port Setup Tab	28

Clock Outputs Tab	29

PLL1 Tab	32

Setting the PLL1 VCO Frequency and PLL2 Reference Frequency	33

PLL2 Tab	34

Bits/Pins Tab	35

Registers Tab	40

Appendix B: Typical Phase Noise Performance Plots	41

PLL1	41

122.88 MHz VCXO Phase Noise	41

Clock Output Measurement Technique	42

Buffered OSCout Phase Noise	42

Clock Outputs (CLKout)	43

LMK04808B CLKout Phase Noise	43

LMK04808B OSCout Phase Noise	45

LMK04806B CLKout Phase Noise	46

LMK04806B OSCout Phase Noise	48

LMK04803B CLKout Phase Noise	49

LMK04803B OSCout Phase Noise	51

Appendix C: Schematics	52

Power Supplies	52

LMK048xxB Device with Loop Filter and Crystal Circuits	53

Reference Inputs (CLKin0 & CLKin1), External VCXO (OSCin) & VCO Circuits, and OSCout1 Output	54

Clock Outputs (OSCout0, CLKout0 to CLKout3) 	55

Clock Outputs (CLKout4 to CLKout7)	56

Clock Outputs (CLKout8 to CLKout11)	57

uWire Header, Logic I/O Ports and Status LEDs	58

USB Interface	59

Appendix D: Bill of Materials	60

Appendix E:  PCB Layers Stackup	70

Appendix F: PCB Layout	71

Layer #1 – Top	71

Layer #2 – RF Ground Plane (Inverted)	72

Layer #3 – Vcc Planes	73

Layer #4 – Ground Plane (Inverted)	74

Layer # 5 – Vcc Planes 2	75

Layer #6 – Bottom	76

Layers #1 and 6 – Top and Bottom (Composite)	77

Appendix G: EVM Software and Communication: Interfacing uWire	78

Appendix H: Troubleshooting Information	80

1)	Confirm Communications	80

2)	Confirm PLL1 operation/locking	80

3)	Confirm PLL2 operation/locking	81

# General Description

The LMK048xx Evaluation Board simplifies evaluation of the LMK048xxB Low-Noise Clock Jitter Cleaner with Dual Loop PLLs.  Texas Instruments Incorporated’s CodeLoader software can be used to program the internal registers of the LMK048xxB device through the USB2ANY-uWire interface.  The CodeLoader software will run on a Windows 7 or Windows XP PC and can be downloaded from http://www.ti.com/tool/codeloader/.

## Evaluation Board Kit Contents

The evaluation board kit includes:

- (1) LMK048xx Evaluation Board from Table 1

- (1) LMK04800 Family Quick Start Guide

  - Evaluation board instructions can be downloaded from www.ti.com.

- (1) LPT Cable or USB2ANY-uWire (HPA665-001) + adapter (Please see “EVM Software and Communication” for more information)

## Available LMK048xx Evaluation Boards

The LMK048xx Evaluation Board supports any of the four devices offered in the LMK04800 Family.  All evaluation boards use the same PCB layout and bill-of-materials, except for the corresponding LMK0480xxB device affixed to the board.  A commercial-quality VCXO is also mounted to the board to provide a known reference point for evaluating device performance and functionality.

Table 1: Available Evaluation Board Configurations

| Evaluation Board ID | Device | PLL1 VCXO |
| --- | --- | --- |
| LMK04803BEVAL | LMK04803B | 122.88 MHz Crystek VCXO Model CVHD-950-122.880 |
| LMK04805BEVAL | LMK04805B |  |
| LMK04806BEVAL | LMK04806B |  |
| LMK04808BEVAL | LMK04808B |  |

## Available LMK04800 Family Devices

Table 2: LMK048xxB Devices

| Device | Reference Inputs | Buffered/ Divided OSCin Outputs | Programmable LVDS/LVPECL/ LVCMOS Outputs | VCO Frequency |
| --- | --- | --- | --- | --- |
| LMK04803B | 2 | 2 | 12 | 1840 to 2030 MHz |
| LMK04805B |  |  |  | 2148 to 2370 MHz |
| LMK04806B |  |  |  | 2370 to 2600 MHz |
| LMK04808B |  |  |  | 2750 to 3072 MHz |

# Quick Start

Full evaluation board instructions are downloadable from the LMK048xxB device product folder at www.ti.com.

- Connect a power supply voltage of 5 V to the Vcc SMA connector.  The onboard LP3878-ADJ LDO regulator will output a low-noise 3.3 V supply to operate the device.

- Connect a reference clock from a signal source to the CLKin1 SMA port.  Use 122.88 MHz for default.  The reference frequency depends on the device programming.

- Please see Appendix G for quick start on interfacing the board

- Program the device with a default mode using CodeLoader.  Ctrl+L must be pressed at least once to load all registers.  Alternatively click menu “Keyboard Controls”  “Load Device”.  CodeLoader can be downloaded from http://www.ti.com/tool/codeloader.

- Measurements may be made on an active output clock port via its SMA connector.

Figure 1: Quick Start Diagram

## Default CodeLoader Modes for Evaluation Boards

CodeLoader saves the state of the selected LMK048xxB device when exiting the software.  To ensure a common starting point, the following modes listed in Table 3 may be restored by clicking “Mode” and selecting the appropriate device configuration, as shown in Figure 2 in the case of the LMK04808B device.  Similar default modes are available for each LMK048xxB device in CodeLoader. Choose a mode with CLKin0 for differential clock signal or CLKin1 for a single ended signal.

Figure 2: Selecting a Default Mode for the LMK04808 Device

After restoring a default mode, press Ctrl+L to program the device.  The default modes also disable certain outputs, so make sure to enable the output under test to make measurements.

Table 3: Default CodeLoader Modes for LMK04808

| Default CodeLoader Mode | Device Mode | CLKin Frequency | OSCin Frequency |
| --- | --- | --- | --- |
| 122.88 MHz CLKin1, 122.88 MHz VCXO | Dual PLL, Internal VCO | 122.88 MHz | 122.88 MHz |
| 122.88 MHz CLKin1, Dual Loop 0-delay, 122.88 MHz VCXO | Dual PLL, Internal VCO, 0-Delay with Internal Feedback | 122.88 MHz | 122.88 MHz |
| 122.88 MHz CLKin1, 122.88 MHz VCXO | Dual PLL, Internal VCO, PLL2 Crystal Oscillator Enabled | 122.88 MHz | 20.48 MHz |
| 122.88 MHz CLKin1, 122.88 MHz VCXO | Dual PLL, Internal VCO | 122.88 MHz | 122.88 MHz |
| 122.88 MHz CLKin1, Dual Loop 0-delay, 122.88 MHz VCXO | Dual PLL, Internal VCO, 0-Delay with Internal Feedback | 122.88 MHz | 122.88 MHz |
| 122.88 MHz CLKin1, 122.88 MHz VCXO | Dual PLL, Internal VCO, PLL2 Crystal Oscillator Enabled | 122.88 MHz | 20.48 MHz |

The next section outlines step-by-step procedures for using the evaluation board with the LMK04808B.  For boards with another part number, make sure to select the corresponding part number under the “Device” menu.

# Example: Using CodeLoader to Program the LMK04808B

The purpose of this section is to walk the user through using CodeLoader 4 to make some measurements with the LMK04808B device as an example.  For more information on CodeLoader refer to Appendix A: CodeLoader Usage or the CodeLoader 4 instructions located at http://www.ti.com/tool/codeloader.

Before proceeding, be sure to follow the Quick Start section above to ensure proper connections.

## 1. Start CodeLoader 4 Application

Click “Start”  “Programs”  “CodeLoader 4”  “CodeLoader 4”

The CodeLoader 4 program is installed by default to the CodeLoader 4 application group.

## 2. Select Device

Click “Select Device”  “Clock Conditioners”  “LMK04808B”

Once started CodeLoader 4 will load the last used device.  To load a new device click “Select Device” from the menu bar, then select the subgroup and finally device to load.  For this example, the LMK04808B is chosen.  Selecting the device does cause the device to be programmed.

Figure 3 – Selecting the LMK04808B device

3. Program/Load Device

Figure 4 – Loading the Device

Assuming the Port Setup settings are correct, press the “Ctrl+L” shortcut or click “Keyboard Controls”  “Load Device” from the menu to program the device to the current state of the newly loaded LMK04800 file.

Once the device has been initially loaded, CodeLoader will automatically program changed registers so it is not necessary to re-load the device upon subsequent changes in the device configuration.  It is possible to disable this functionality by ensuring there is no checkmark by the “Options”  “AutoReload with Changes.”

Because a default mode will be restored in the next step, this step isn’t really needed but included to emphasize the importance of pressing “Ctrl+L” to load the device at least once after starting CodeLoader, restoring a mode, or restoring a saved setup using the File menu.

See Appendix A: CodeLoader Usage or the CodeLoader 4 instructions located at http://www.ti.com/tool/codeloader for more information on Port Setup.  Appendix H: Troubleshooting Information contains information on troubleshooting communications.

## 4. Restoring a Default Mode

Click “Mode”  “LMK04808B, 122.88 MHz VCXO, 122.88 MHz CLKin1”; then press Ctrl+L.

Figure 5: Setting the Default mode for LMK04808

For the purpose of this walkthrough, a default mode will be loaded to ensure a common starting point.  This is important because when CodeLoader is closed, it remembers the last settings used for a particular device.  Again, remember to press Ctrl+L as the first step after loading a default mode.

## 5. Visual Confirmation of Frequency Lock

After a default mode is restored and loaded, LED D5 should illuminate when PLL1 and PLL2 are locked to the reference clock applied to CLKin1.  This assumes LD_MUX = PLL1/2 DLD and LD_TYPE = Active High, which are the default settings.

## 6. Enable Clock Outputs

While the LMK048xxB offers programmable clock output buffer formats, the evaluation board is shipped with preconfigured output terminations to match the default buffer type for each output.  Refer to the CLKout port description in the Evaluation Board Inputs and Outputs section.

To measure phase noise at one of the clock outputs, for example, CLKout0:

- Click on the Clock Outputs tab,

- Uncheck “Powerdown” in the Digital Delay box to enable the channel,

- Set the following settings as needed:

  - Digital Delay value

  - Clock Divider value

  - Analog Delay select and Analog Delay value (if not “Bypassed”)

  - Clock Output type.

Figure 6: Setting Digital Delay, Clock Divider, Analog Delay, and Output Format for CLKout0

- Depending on the configured output type, the clock output SMAs can be interfaced to a test instrument with a single-ended 50-ohm input as follows.

  - For LVDS:

    - A balun (like ADT2-1T) is recommended for differential-to-single-ended conversion.

  - For LVPECL:

    - A balun can be used, or

    - One side of the LVPECL signal can be terminated with a 50-ohm load and the other side can be run single-ended to the instrument.

  - For LVCMOS:

    - There are two single-ended outputs, CLKoutX and CLKoutX*, and each output can be set to Normal, Inverted, or Off.  There are nine (9) combinations of LVCMOS modes in the Clock Output list.

    - One side of the LVCMOS signal can be terminated with a 50-ohm load and the other side can be run single-ended to the instrument.

Figure 7: Setting LVCMOS modes

A balun may also be used.  Ensure CLKoutX and CLKoutX* states are complementary to each other, i.e.: Norm/Inv or Inv/Norm.

- The phase noise may be measured with a spectrum analyzer or signal source analyzer.

See Appendix B: Typical Phase Noise Performance Plots for phase noise plots of the clock outputs.

Texas Instruments Incorporated’s Clock Design Tool can be used to calculate divider values to achieve desired clock output frequencies.  See: http://www.ti.com/tool/clockdesigntool.

# PLL Loop Filters and Loop Parameters

In jitter cleaning applications that use a cascaded or dual PLL architecture, the first PLL’s purpose is to substitute the phase noise of a low-noise oscillator (VCXO or crystal resonator) for the phase noise of a “dirty” reference clock.  The first PLL is typically configured with a narrow loop bandwidth in order to minimize the impact of the reference clock phase noise. The reference clock consequently serves only as a frequency reference rather than a phase reference.

The loop filters on the LMK048xx evaluation board are setup using the approach above. The loop filter for PLL1 has been configured for a narrow loop bandwidth (< 100 Hz), while the loop filter of PLL2 has been configured for a wide loop bandwidth (> 100 kHz). The specific loop bandwidth values depend on the phase noise performance of the oscillator mounted on the board. The following tables contain the parameters for PLL1 and PLL2 for each oscillator option.

Texas Instruments Incorporated’s Clock Design Tool can be used to optimize PLL phase noise/jitter for given specifications.  See: http://www.ti.com/tool/clockdesigntool/.

## PLL 1 Loop Filter

Table 4: PLL1 Loop Filter Parameters for Crystek 122.88 MHz VCXO

| ### 122.88 MHz VCXO PLL |  |  |  |
| --- | --- | --- | --- |
| Phase Margin | 49˚ | Kφ (Charge Pump) | 100 uA |
| Loop Bandwidth | 12 Hz | Phase Detector Freq | 1.024 MHz |
|  |  | VCO Gain | 2.5 kHz/Volt |
| Reference Clock Frequency | 122.88 MHz | Output Frequency | 122.88 MHz (To PLL 2) |
| Loop Filter Components | C1_A1 = 100 nF | C2_A1 = 680 nF | R2_A1 = 39 kΩ |

Note: PLL Loop Bandwidth is a function of K, Kvco, N as well as loop components. Changing K and N will change the loop bandwidth.

## PLL2 Loop Filter

Table 5: PLL2 Loop Filter Parameters for LMK048xxB

| ### Integrated VCO PLL |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | LMK04803B | LMK04805B | LMK04806B | LMK04808B |  |
| C1_A2 | 0.047 |  |  |  | nF |
| C2_A2 | 3.9 |  |  |  | nF |
| C3 (internal) | 0 |  |  |  | nF |
| C4 (internal) | 0 |  |  |  | nF |
| R2_A2 | 0.62 |  |  |  | kΩ |
| R3 (internal) | 0.2 |  |  |  | kΩ |
| R4 (internal) | 0.2 |  |  |  | kΩ |
| Charge Pump Current, K | 3.2 |  |  |  | mA |
| Phase Detector Frequency | 122.88 |  |  |  | MHz |
| Frequency | 1966.08 | 2211.84 | 2457.6 | 2949.12 | MHz |
| Kvco | 17.9 | 16.5 | 18.8 | 32.3 | MHz/V |
| N | 16 | 18 | 20 | 24 |  |
| Phase Margin | 75 | 75 | 75 | 76 | degrees |
| Loop Bandwidth | 355 | 320 | 321 | 424 | kHz |

Note: PLL Loop Bandwidth is a function of K, Kvco, N as well as loop components. Changing K and N will change the loop bandwidth.

# Evaluation Board Inputs and Outputs

The following table contains descriptions of the inputs and outputs for the evaluation board.  Unless otherwise noted, the connectors described can be assumed to be populated by default.  Additionally, some applicable CodeLoader programming controls are noted for convenience.  Refer to the LMK04800 Family Datasheet for complete register programming information.

Table 6: Evaluation Board Inputs and Outputs

| Connector Name | Signal Type, Input/Output | Description |
| --- | --- | --- |
| Populated: CLKout0, CLKout0*, CLKout2, CLKout2*, CLKout4, CLKout4*, CLKout6, CLKout6*, CLKout8, CLKout8*, CLKout10, CLKout10* Not Populated: CLKout1, CLKout1*, CLKout3, CLKout3*, CLKout5, CLKout5*, CLKout7, CLKout7*, CLKout9, CLKout9*, CLKout11, CLKout11* | Analog, Output | Clock outputs with programmable output buffers. The output terminations by default on the evaluation board are shown below, and the output type selected by default in CodeLoader is indicated by an asterisk (*): Clock output pair; Default Board Termination. CLKout0; LVPECL*. CLKout1; LVPECL. CLKout2; LVPECL*. CLKout3; LVPECL. CLKout4; LVDS* / LVCMOS. CLKout5; LVDS / LVCMOS. CLKout6; LVDS* / LVCMOS. CLKout7; LVDS / LVCMOS. CLKout8; LVDS* / LVCMOS. CLKout9; LVDS / LVCMOS. CLKout10; LVPECL*. CLKout11; LVPECL. Each CLKout pair has a programmable LVDS, LVPECL, or LVCMOS buffer. The output buffer type can be selected in CodeLoader in the Clock Outputs tab via the CLKoutX_TYPE control. All clock outputs are AC-coupled to allow safe testing with RF test equipment. All LVPECL clock outputs are source-terminated using 240-ohm resistors. If an output pair is programmed to LVCMOS, each output can be independently configured (normal, inverted, or off/tri-state). |
| Populated: OSCout0, OSCout0*, OSCout1, OSCout1* | Analog, Output | Buffered outputs of OSCin port. The output terminations on the evaluation board are shown below, the output type selected by default in CodeLoader is indicated by an asterisk (*): OSC output pair; Default Board Termination. OSCout0; LVDS* / LVCMOS. OSCout1; LVPECL* (fixed). Only OSCout0 has a programmable LVDS, LVPECL, or LVCMOS output buffer. The OSCout0 buffer type can be selected in CodeLoader on the Clock Outputs tab via the OSCout0_TYPE control. OSCout1 has LVPECL buffer only but has programmable swing amplitude. Both OSCout pairs are AC-coupled to allow safe testing with RF test equipment. The OSCout1 output is source-terminated using 240-ohm resistors. If OSCout0 is programmed as LVCMOS, each output can be independently configured (normal, inverted, inverted, and off/tri-state). |
| Vcc | Power, Input | Main power supply input for the evaluation board. A 3.9 V DC power source applied to this SMA will, by default, source the onboard LDO regulators that power the inner layer planes that supply the LMK048xxB and its auxiliary circuits (e.g. VCXO). The LMK048xxB contains internal voltage regulators for the VCO, PLL and other internal blocks. The clock outputs do not have an internal regulator, so a clean power supply with sufficient output current capability is required for optimal performance. On-board LDO regulators and 0  resistor options provide flexibility to supply and route power to various devices. See schematics for more details. |
| Populated: J1 | Power, Input | Alternative power supply input for the evaluation board using two unshielded wires (Vcc and GND). Apply power to either Vcc SMA or J1, but not both. |
| Populated: VccVCO/Aux | Power, Input | Optional Vcc input to power the VCO circuit if separated voltage rails are needed. The VccVCO/Aux input can power these circuits directly or supply the on-board LDO regulators. 0 Ω resistor options provide flexibility to route power. |
| Populated: VccVCXO/Aux | Power, Input | Optional Vcc input to power the VCXO circuit if separated voltage rails are needed. The VccVCXO/Aux input can power these circuits directly or supply the on-board LDO regulators. 0 Ω resistor options provide flexibility to route power. |
| Populated: CLKin0, CLKin0*, FBCLKin*/CLKin1* Not Populated: FBCLKin/CLKin1 | Analog, Input | Reference Clock Inputs for PLL1 (CLKin0, 1). CLKin1 can alternatively be used as an External Feedback Clock Input (FBCLKin) in 0-delay mode or an RF Input (Fin) in External VCO mode. Reference Clock Inputs for PLL1 (CLKin0, 1) FBCLKin/CLKin1* is configured by default for a single-ended reference clock input from a 50-ohm source. The non-driven input pin (FBCLKin/CLKin1) is connected to GND with a 0.1 uF. CLKin0/CLKin0* is configured by default for a differential reference clock input from a 50-ohm source. CLKin1* is the default reference clock input selected in CodeLoader. The clock input selection mode can be programmed on the Bits/Pins tab via the CLKin_Select_MODE control. Refer to the LMK04800 Family Datasheet section “Input Clock Switching” for more information. AC coupled Input Clock Swing Levels Input; Mode; Min; Max; Units. Differential; Bipolar or CMOS; 0.5; 3.1; Vpp. Single Ended; 0.25; 2.4; Vpp. External Feedback Input (FBCLKin) for 0-Delay CLKin1 is shared for use with FBCLKin as an external feedback clock input to PLL1 for 0-delay mode. See section, Programming 0-Delay Mode in CodeLoader below, for more details on using 0-delay mode with the evaluation board and the evaluation board software. RF Input (Fin) for External VCO CLKin1 is also shared for use with Fin as an RF input for external VCO mode using the onboard VCO footprint (U3) or add-on VCO board. To enable Dual PLL mode with External VCO, the following registers must be properly configured in CodeLoader: - MODE = (3) Dual PLL, Ext VCO (Fin), (5) Dual PLL, Ext VCO, 0-Delay, (11) PLL2, Ext VCO (Fin) |
| Not populated: OSCin, OSCin* | Analog, Input | Feedback VCXO clock input to PLL1 and Reference clock input to PLL2. By default, these SMAs are not connected to the traces going to the OSCin/OSCin* pins of the LMK048xxB. Instead, the single-ended output of the onboard VCXO (U2) drives the OSCin* input of the device and the OSCin input of the device is connected to GND with 0.1 uF. A VCXO add-on board may be optionally attached via these SMA connectors with minor modification to the components going to the OSCin/OSCin* pins of device. This is useful if the VCXO footprint does not accommodate the desired VCXO device. A single-ended or differential signal may be used to drive the OSCin/OSCin* pins and must be AC coupled. If operated in single-ended mode, the unused input must be connected to GND with 0.1 uF. Refer to the LMK04800 Family Datasheet section “Electrical Characteristics” for PLL2 Reference Input (OSCin) specifications. |
| Test point: VTUNE1_TP Not populated: Vtune1 | Analog, Output | Tuning voltage output from the loop filter for PLL1. If a VCXO add-on board is used, this tuning voltage can be connected to the voltage control pin of the external VCXO when this SMA connector is installed and connected through R72 by the user. |
| Test point: VTUNE2_TP | Analog, Output | Tuning voltage output from the loop filter for PLL2. |
| Populated: uWire Test points: DATAuWire_TP CLKuWIRE_TP LEuWIRE_TP | CMOS, Input/Output | 10-pin header for uWire programming interface and programmable logic I/O pins for the LMK048xxB. The uWire interface includes CLKuWire, DATAuWire, and LEuWire signals. The programmable logic I/O signals accessible through this header include: SYNC, Status_Holdover, Status_LD, Status_CLKin0, and Status_CLKin1. These logic I/O signals also have dedicated SMAs and test points. |
| Test point: LD_TP Not populated: Status_LD | CMOS, Output | Programmable status output pin. By default, set to output the digital lock detect status signal for PLL1 and PLL2 combined. In the default CodeLoader modes, LED D5 will illuminate green when PLL lock is detected by the LMK048xxB (output is high) and turn off when lock is lost (output is low). The status output signal for the Status_LD pin can be selected on the Bits/Pins tab via the LD_MUX control. Refer to the LMK04800 Family Datasheet section “Status Pins” and “Digital Lock Detect” for more information. Note: Before a high-frequency internal signal (e.g. PLL divider output signal) is selected by LD_MUX, it is suggested to first remove the 270 ohm resistor to prevent the LED from loading the output. |
| Test point: Holdover_TP Not populated: Status_Hold | CMOS, Output | Programmable status output pin. By default, set to the output holdover mode status signal. In the default CodeLoader mode, LED D8 will illuminate red when holdover mode is active (output is high) and turn off when holdover mode is not active (output is low). The status output signal for the Status_Holdover pin can be selected on the Bits/Pins tab via the HOLDOVER_MUX control. Refer to the LMK04800 Family Datasheet section “Status Pins” and “Holdover Mode” for more information. Note: Before a high-frequency internal signal (e.g. PLL divider output signal) is selected by HOLDOVER_MUX, it is suggested to first remove the 270 ohm resistor to prevent the LED from loading the output. |
| Test point: CLKin0_SEL_TP CLKin1_SEL_TP Not populated: Status_CLKin0, Status_CLKin1 | CMOS, Input/Output | Programmable status I/O pins. By default, set as input pins for controlling input clock switching of CLKin0 and CLKin1. These inputs will not be functional because CLKin_Select_MODE is set to 0 (CLKin0 Manual) by default in the Bits/Pins tab in CodeLoader. To enable input clock switching, CLKin_Select_MODE must be 3 or 6 and Status_CLKinX_TYPE must be 0 to 3 (pin enabled as an input). Input Clock Switching – Pin Select Mode When CLKin_SELECT_MODE is 3, the Status_CLKinX pins select which clock input is active as follows: Status_CLKin1; Status_CLKin0; Active Clock. 0; 0; CLKin0. 0; 1; CLKin1. 1; 0; Reserved. 1; 1; Holdover. Input Clock Switching – Auto with Pin Select When CLKin_SELECT_MODE is 6, the active clock is selected using the Status_CLKinX pins upon an input clock switch event as follows: Status_CLKin1; Status_CLKin0; Active Clock. X; 0; CLKin0. 1; 0; CLKin1. 0; 0; Reserved. Refer to the LMK04800 Family Datasheet section “Input Clock Switching” for more information. Status Outputs When Status_CLKinX_TYPE is 3 to 6 (pin enabled as an output), the status output signal for the corresponding Status_CLKinX pin can be programmed on the Bits/Pins tab via the Status_CLKinX_MUX control. Refer to the LMK04800 Family Datasheet section “Status Pins” for more information. |
| Test point: SYNC_TP Not populated: SYNC | CMOS, Input/Output | Programmable status I/O pin. By default, set as an input pin for synchronize the clock outputs with a fixed and known phase relationship between each clock output selected for SYNC. A SYNC event also causes the digital delay values to take effect. In the default CodeLoader mode, SYNC will asserted when the SYNC pin is low and the outputs to be synchronized will be held in a logic low state. When SYNC is unasserted, the clock outputs to be synchronized are activated and will be initially phase aligned with each other except for outputs programmed with different digital delay values. A SYNC event can also be programmed by toggling the SYNC_POL_INV bit in the Bits/Pins tab in CodeLoader. Refer to the LMK04800 Family Datasheet section “Clock Output Synchronization” for more information. Status Output When SYNC_MUX is 3 to 6 (pin enabled as output), a status signal for the SYNC pin can be selected on the Bits/Pins tab via the SYNC_MUX control. |

# Recommended Test Equipment

Power Supply

The Power Supply should be a low noise power supply, particularly when the devices on the board are being directly powered (onboard LDO regulators bypassed).

Phase Noise / Spectrum Analyzer

To measure phase noise and RMS jitter, an Agilent E5052 Signal Source Analyzer is recommended.  An Agilent E4445A PSA Spectrum Analyzer with the Phase Noise option is also usable although the architecture of the E5052 is superior for phase noise measurements.  At frequencies less than 100 MHz the local oscillator noise of the E4445A is too high and measurements will reflect the E4445A’s internal local oscillator performance, not the device under test.

Oscilloscope

To measure the output clocks for AC performance, such as rise time or fall time, propagation delay, or skew, it is suggested to use a real-time oscilloscope with at least 1 GHz analog input bandwidth (2.5+ GHz recommended) with 50 ohm inputs and 10+ Gsps sample rate.  To evaluate clock synchronization or phase alignment between multiple clock outputs, it’s recommended to use phase-matched, 50-ohm cables to minimize external sources of skew or other errors/distortion that may be introduced if using oscilloscope probes.

# Programming 0-Delay Mode in CodeLoader

## Overview

When enabling the 0-Delay mode the feedback path of the VCO is altered to include a clock output.  See the datasheet for more details on 0-Delay functionality.

The current version of the CodeLoader software does not include this extra divider in the frequency calculations when in holdover mode.  To successfully lock the LMK04800 device in a 0-Delay mode the user must program the device “manually” account for this divider.  Programming “manually” means that the VCO frequency and therefore the clock output frequencies displayed by the CodeLoader software may be incorrect.  For the LMK04800 device to lock properly the divider values must be programmed correctly.  The frequencies displayed in the application are only for the benefit of the user and for proper automatic programming of the OSCin_FREQ register which will not be affected by 0-Delay.

When using the device in Dual Loop mode vs. Single Loop mode different procedures are used to cause the device to lock when using the CodeLoader software.  The following two sections describe the process for when the LMK04800 device is programmed for a Dual Loop mode and Single Loop mode respectively.  Each section contains a brief introduction, the programming steps to execute to make the device lock, and finally a detailed section discussing the workaround and some example cases.

## Dual Loop 0-Delay Mode Examples

In Dual Loop 0-Delay Modes, MODE = 2 or MODE = 5, the feedback from the VCXO of PLL1 to the PLL1 N divider is broken and a clock output will drive the PLL1 N divider.  This permits phase alignment between the clock output and the clock input (0-Delay).  As such, the PLL1_N and PLL1_R divide values may need to be adjusted to permit the LMK04800 to lock.

### Programming Steps

- Program a Dual Loop 0-Delay mode.

- Enable the feedback mux.  EN_FEEDBACK_MUX = 1.

- Select clock output for feedback with the feedback mux.  FEEDBACK_MUX = User value.

- Program the VCXO (VCO) frequency of PLL1 tab to the clock output frequency selected by the feedback mux.

If for any reason the CLKout frequency is less than the phase detector frequency, the PLL1 R divider must be increased so that the phase detector is at the same or lower value than the CLKout frequency.

### Details

When using the CodeLoader software in Dual Loop 0-Delay mode, programming the VCXO (VCO) frequency of the PLL1 tab to the frequency of the fed back output clock will re-program the PLL1 N divider to allow the LMK04800 will be able to lock.  The PLL1 loop has been altered and actual VCXO no longer directly feeds into PLL1 N divider.  The VCXO is only used by the reference input of PLL2 now.  The PLL2 reference frequency will remain at the VCXO frequency.

When the PLL1 VCXO frequency is different from the PLL2 reference frequency, a warning will be displayed on the clock outputs tab informing the user that PLL1 VCO and PLL2 reference frequency are mismatched and the one or more of the PLLs are out of lock.  While there still could be an error in the divider values which may cause a non-locked PLL, this warning by itself may no longer be assumed true.  It is up to the user to ensure the PLL dividers are programmed correctly.

To illustrate the proper programming of the LMK04800 device in dual loop 0-delay mode the following case examples are provided.  Note that in one of the cases, the feedback frequency from the clock output matches the VCXO frequency and CodeLoader will display the proper frequency values.

Dual Loop 0-Delay (MODE=2 or 5) Case 1: For example the default configuration, 122.88 MHz CLKin, 122.88 MHz VCXO, of the LMK04808 has the following register programming.

|  | Case 1: Default Mode No 0-Delay | Case2: Default 0-Delay Mode (CLKout8 = 122.88 MHz) | Case 3: Default 0-Delay Mode (Updated CLKout8 = 245.76 MHz) | Case 4: Default 0-Delay Mode (Updated CLKout8 = 61.44 MHz) |
| --- | --- | --- | --- | --- |
| Actual PLL1 VCXO Frequency | 122.88 | 122.88 | 122.88 | 122.88 |
| Reported PLL1 VCXO Frequency | 122.88 | 122.88 | 61.44 | 245.76 |
| PLL1 N | 120 | 120 | 60 | 240 |
| Actual PLL2 VCO Frequency | 2949.12 MHz | 2949.12 MHz | 2949.12 MHz | 2949.12 MHz |
| Reported PLL2 VCO Frequency | 2949.12 MHz | 2949.12 MHz | 2949.12 MHz | 2949.12 MHz |
| PLL2_N | 12 | 12 | 12 | 12 |
| PLL2_P (Pre-N) | 2 | 2 | 2 | 2 |
| PLL2 VCO Divider | Bypassed | Bypassed | Bypassed | Bypassed |
| CLKout8 Divide | 12 | 24 | 12 | 48 |
| Actual CLKout8 Output Frequency | 245.76 MHz | 122.88 MHz | 245.76 MHz | 61.44 MHz |
| Reported CLKotu8 Output Frequency | 245.76 MHz | 122.88 MHz | 245.76 MHz | 61.44 MHz |

## Single Loop 0-Delay Mode Examples

In Single Loop 0-Delay Mode, MODE = 8, the feedback from the VCO of PLL2 to the PLL2_P/PLL2 N divider is broken and a fed back clock output will drive the PLL2 N divider directly.  This permits phase alignment between the clock output and the OSCin input (0-Delay).  As such, the PLL2_N, PLL2_R, and PLL2_N_CAL divide values may need to be adjusted to permit the LMK04800 to lock.

### Programming Steps

- Program the Single Loop 0-Delay mode.

- Enable the feedback mux.  EN_FEEDBACK_MUX = 1.

- Select clock output for feedback with the feedback mux.  FEEDBACK_MUX = User value.

- Program the VCO frequency of PLL2 tab to: The actual VCO frequency * PLL2_P (which is PLL2 PreN) / CLKout Divider.

  - Entered CodeLoader 4 VCO Frequency = Actual VCO Frequency * PLL2_P / CLKout Divider.

- Updated the PLL2_N_CAL register on the Bits/Pins tab to the N value when in non-0-Delay mode.

- Press Ctrl-L to cause all registers to be programmed.

  - The reason is to cause the programming of register R30 to start the VCO calibration routine now that the proper PLL2_N_CAL value is programmed.

  - PLL2_N_CAL value is automatically updated when a new VCO frequency is entered and the PLL2_N value is calculated.  In this case the VCO frequency entered is wrong and the PLL2_N_CAL value will be incorrect.

If for any reason the CLKout frequency is less than the phase detector frequency, the PLL2 R divider must be increased so that the phase detector is at the same or lower value than the CLKout frequency.

### Details

The 0-Delay mode for Single Loop mode is more complicated to program than for Dual Loop mode in part because of the PLL2_N_CAL register.  When performing the VCO calibration the device uses PLL2_N_CAL for in non-0-Delay mode.  Once the VCO is calibrated the device enters 0-Delay mode.  For more information on the PLL programming equations, refer to PLL PROGRAMMING in the applications section of the datasheet.

In Table 7 case 1 illustrates the register programming when note using 0-Delay.

Case 2 shows 0-Delay with a clock out divider of 2.  Since PLL2_P = 2, this substitution of which circuit is performing the divide by two results in no impact o the software.  All the values display correctly.

Case 3 shows 0-Delay mode with a CLKout divider not equal to the PLL2_P value.  So the proper frequency to program in the VCO to lock the VCO to 2949.12 MHz will be 491.52 MHz.  This is calculated by Actual VCO Frequency * PLL2_P / CLKoutX_Y_DIV.

Case 4 shows 0-Delay mode with CLKout divider not equal to the PLL2_P value; however the CLKout frequency will be less than the current phase detector frequency.  This requires PLL2_R to be increased from a value of 1 to 2 to reduce the PLL2 phase detector frequency from 122.88 MHz to 61.44 MHz.  Now the adjusted VCO frequency can be programmed to allow PLL2 to lock.

In any case where the actual VCO frequency and the display VCO frequency are not equal the user is required to manually update the PLL2_N_CAL register with the PLL2_N value to be used as if the device were operating in the non-0-Delay mode.  Once this update has been performed, Ctrl-L will reload the part and cause the VCO calibration to occur with the proper PLL2_N_CAL value.

Table 7 - Single PLL 0-Delay Operation Examples

|  | Case 1: Default Mode No 0-Delay | Case 2: Default 0-Delay Mode (CLKout8 = 1474.56 MHz) | Case 3: Default 0-Delay Mode (Updated CLKout8 = 245.76 MHz) | Case 4: Default 0-Delay Mode (Updated CLKout8 = 61.44 MHz) |
| --- | --- | --- | --- | --- |
| Actual PLL2 VCO Frequency | 2949.12 MHz | 2949.12 MHz | 2949.12 MHz | 2949.12 MHz |
| Reported PLL2 VCO Frequency | 2949.12 MHz | 2949.12 MHz | 491.52 MHz | 122.88 MHz |
| PLL2_R | 1 | 1 | 1 | 2 |
| PLL2_N | 12 | 12 | 2 | 1 |
| PLL2_N_CAL | 12 | 12 | 12 | 24 |
| PLL2_P (Pre-N) | 2 | 2 | 2 | 2 |
| PLL2 VCO Divider | Bypassed | Bypassed | Bypassed | Bypassed |
| CLKout8 Divide | 12 | 2 | 12 | 48 |
| Actual CLKout8 Output Frequency | 245.76 | 1474.56 MHz | 245.76 MHz | 61.44 MHz |
| Reported CLKout8 Output Frequency | 245.76 | 1474.56 MHz | 40.96 MHz | 2.56 MHz |

# Appendix A: CodeLoader Usage

Code Loader is used to program the evaluation board with LPT or USB2ANY-uWire interface available from www.ti.com.

## Port Setup Tab

Figure 8: Port Setup tab

On the Port Setup tab, the user may select the type of communication port (LPT or USB) that will be used to program the device on the evaluation board.

The Pin Configuration field is hardware dependent and normally does not need to be changed by the user.  Figure 8 shows the default settings.

## Clock Outputs Tab

Figure 9: Clock Outputs tab

The Clock Outputs tab allows the user to control the output channel blocks, including:

- Clock Group Source from either VCO or OSCin (via OSC Mux1 and OSC Mux2)

- Channel Powerdown (affects digital and analog delay, clock divider, and buffer blocks)

- Digital Delay value and Half Step

- Clock Divide value

- Analog Delay value and Delay bypass/enable (per output)

- Clock Output format (per output)

This tab also allows the user to select the VCO Divider value (2 to 8).  Note that the total PLL2 N divider value is the product of the VCO Divider value and the PLL N Prescaler and N Counter values (shown in the PLL2 tab), and is given by:

PLL2 N Total = VCO Divider * PLL2 N Prescaler * PLL2 N Counter

Clicking on the cyan-colored PLL2 block that contains R, PDF and N values will bring the PLL2 tab into focus where these values may be modified, if needed.

Clicking on the values in the box containing the Internal Loop Filter component (R3, C3, R4, C4) allow one to step through the possible values.  Left click to increase the component value, and right click to decrease the value.  These values can also be changed in the Bits/Pins tab.

The Reference Oscillator value field may be changed in either the Clock Outputs tab or the PLL2 tab.  The PLL2 Reference frequency should match the frequency of the onboard VCXO or Crystal (i.e. VCO frequency in the PLL1 tab); if not, a warning message will appear to indicate that the PLL(s) may be out of lock, as highlighted by the red box in Figure 10.

Figure 10: Warning message indicating mismatch between 
PLL1 VCO frequency (30.72MHz) and PLL2 reference frequency (122.88 MHz)

## PLL1 Tab

Figure 11: PLL1 tab

The PLL1 tab allows the user to change the following parameters in Table 8.

Table 8: Registers Controls and Descriptions in PLL1 tab

| Control Name | Register Name | Description |
| --- | --- | --- |
| Reference Oscillator Frequency (MHz) | n/a | CLKin frequency of the selected reference clock. |
| Phase Detector Frequency (MHz) | n/a | PLL1 Phase Detector Frequency (PDF). This value is calculated as: PLL1 PDF = CLKin Frequency / (PLL1_R * CLKinX_PreR_DIV), where CLKinX_PreR_DIV is the predivider value of the selected input clock. |
| VCO Frequency (MHz) | n/a | The VCO Frequency should be the OSCin frequency, except when operating in Dual PLL with 0-delay feedback. This value is calculated as: VCO Freq (OSCin freq) = PLL1 PDF * PLL1_N. In Dual PLL mode with 0-delay feedback, the VCO frequency should be set to the feedback clock input frequency. See the section Setting the PLL1 VCO Frequency and PLL2 Reference Frequency for details. |
| R Counter | PLL1_R | PLL1 R Counter value (1 to 16383). |
| N Counter | PLL1_N | PLL1 N Counter value (1 to 16383). |
| Phase Detector Polarity | PLL1_CP_POL | PLL1 Phase Detector Polarity. Click on the polarity sign to toggle polarity “+” or “–”. |
| Charge Pump Gain | PLL1_CP_GAIN | PLL1 Charge Pump Gain. Left-click/right-click to increase/decrease charge pump gain (100, 200, 400, 1600 uA). |
| Charge Pump State | PLL1_CP_TRI | PLL1 Charge Pump State. Click to toggle between Active and Tri-State. |

### Setting the PLL1 VCO Frequency and PLL2 Reference Frequency

When operating in Dual PLL mode without 0-delay feedback, the VCO frequency value on the PLL1 tab must match the Reference Oscillator (OSCin) frequency value on the PLL2 tab; otherwise, the one or both PLLs may be out of lock.  Updating the Reference Oscillator frequency on the PLL2 tab will automatically update the value of OSCin_FREQ on the Bits/Pins tab.

However, when operating in Dual PLL mode with 0-delay feedback, it may be valid for the VCO frequency value on the PLL1 tab to be different from the Reference Oscillator (OSCin) frequency value on the PLL2 tab.   This is because in 0-delay mode, the PLL1 feedback clock is taken from an output clock instead of the OSCin clock.  For example, if the CLKin frequency (to PLL1_R) is 30.72 MHz, the 0-delay feedback clock frequency (to PLL1_N) is 30.72 MHz, and the VCXO frequency is 122.88 MHz, then the VCO frequency value on the PLL1 tab should be 30.72 MHz (0-delay feedback frequency) and the Reference Oscillator frequency value on the PLL2 tab should be 122.88 MHz (VCXO frequency).  Because of the mismatched frequencies, a warning message will indicate this condition on the Clock Outputs tab but may be disregarded in a case like this.

## PLL2 Tab

Figure 12: PLL2 tab

The PLL2 tab allows the user to change the following parameters in Table 9.

Table 9: Registers Controls and Descriptions in PLL2 tab

| Control Name | Register Name | Description |
| --- | --- | --- |
| Reference Oscillator Frequency (MHz) | OSCin_FREQ | OSCin frequency from the External VCXO or Crystal. |
| Phase Detector Frequency (MHz) | n/s | PLL2 Phase Detector Frequency (PDF). This value is calculated as: PLL2 PDF = OSCin Frequency *(2EN_PLL2_REF_2X) / PLL2_R. |
| VCO Frequency (MHz) | n/a | Internal VCO Frequency should be within the allowable range of the LMK048xxB device. This value is calculated as: VCO Frequency = PLL2 PDF * (PLL2_N * PLL2_P * VCO divider value). |
| Doubler | EN_PLL2_REF_2X | PLL2 Doubler. 0 = Bypass Doubler 1 = Enable Doubler |
| R Counter | PLL2_R | PLL2 R Counter value (1 to 4095). |
| N Counter | PLL2_N | PLL2 N Counter value (1 to 262143). |
| PLLN Prescaler | PLL2_P | PLL2 N Prescaler value (2 to 8). |
| Phase Detector Polarity | PLL2_CP_POL | PLL2 Phase Detector Polarity. Click on the polarity sign to toggle polarity “+” or “–”. |
| Charge Pump Gain | PLL2_CP_GAIN | PLL2 Charge Pump Gain. Left-click/right-click to increase/decrease charge pump gain (100, 400, 1600, 3200 uA). |
| Charge Pump State | PLL2_CP_TRI | PLL2 Charge Pump State. Click to toggle between Active and Tri-State. |

Changes made on this tab will be reflected in the Clock Outputs tab.  The VCO Frequency should conform to the specified internal VCO frequency range for the LMK048xxB device (per Table 2).

## Bits/Pins Tab

Figure 13: Bits/Pins tab

The Bits/Pins tab allows the user to program bits directly, many of which are not available on other tabs.  Brief descriptions for the controls on this tab are provided in Table 10 to supplement the datasheet.  Refer to the LMK04800 Family Datasheet for more information.

TIP: Right-clicking any register name in the Bits/Pins tab will display a Help prompt with the register address, data bit location/length, and a brief register description.

Table 10: Register Controls and Descriptions on Bits/Pins tab

| Group | Register Name | Description |
| --- | --- | --- |
| Mode Control | RESET | Resets the device to default register values. RESET must be cleared for normal operation to prevent an unintended reset every time R0 is programmed. |
|  | POWERDOWN | Places the device in powerdown mode. |
|  | MODE | Selects the operating mode (topology) for the LMK048xx device. |
|  | PD_OSCin | Powers down the OSCin buffer. For use in Clock Distribution mode if OSCin path is not used. |
|  | FEEDBACK_MUX | Selects the feedback source for 0-delay mode. |
|  | OSCin_FREQ | Must be set to the OSCin frequency range for PLL2. Used for proper operation of the internal VCO calibration routine. Entering a reference oscillator frequency on PLL2 tab will automatically update OSCin_FREQ to the proper frequency range. |
|  | VCO_MUX | Selects between VCO and VCO divider to drive the clock distribution path. The VCO divider is only valid if MODE is selecting the Internal VCO. |
|  | uWire_LOCK | When checked, no other uWire programming will have effect. Must be unchecked to enable uWire programming of registers R0 to R30. |
| CLKin | CLKin_Select_MODE | Selects operational mode for how the device selects the reference clock for PLL1. |
|  | EN_CLKin1 | Enables CLKin1 as a usable reference input during auto switching mode. |
|  | EN_CLKin0 | Enables CLKin0 as a usable reference input during auto switching mode. |
|  | CLKinX_BUF_TYPE | Selects the CLKinX input buffer to Bipolar (internal 0 mV offset) or MOS (internal 55 mV offset). |
|  | EN_LOS | Enable the Loss-Of-Signal (LOS) detect circuitry. |
|  | LOS_TIMEOUT | Sets the timeout value for the LOS detect circuitry to assert a loss of signal state on a clock input. |
| Crystal | EN_PLL2_XTAL | Enables Crystal Oscillator |
|  | XTAL_LVL | Sets peak amplitude on the tunable crystal. Values listed are for a 20.48 MHz crystal. |
| IO Control | LD_MUX | Sets the selected signal on the Status_LD pin. |
|  | LD_TYPE | Sets I/O pin type on the Status_LD pin. |
|  | HOLDOVER_MUX | Sets the selected signal on the Status_HOLDOVER pin. |
|  | HOLDOVER_TYPE | Sets I/O pin type on the Status_Holdover pin. |
|  | Status_CLKin0 _MUX | Sets the selected signal on the Status_CLKin0 pin. |
|  | Status_CLKin0_TYPE | Sets I/O pin type on the Status_CLKin0 pin. |
|  | Status_CLKin1_MUX | Sets the selected signal on the Status_CLKin1 pin. |
|  | Status_CLKin1_TYPE | Sets I/O pin type on the Status_CLKin1 pin. |
|  | CLKin_Sel_INV | Inverts the Status_CLKin0/1 pin polarity when set to an input type. Significant when CLKin_SELECT_MODE is 3 or 6. |
| IO Control – Sync | SYNC_MUX | Sets the selected signal on the SYNC pin. |
|  | SYNC_TYPE | Sets I/O pin type on the SYNC pin. |
|  | SYNC_POL_INV | Sets polarity on SYNC input to active low when checked. Toggling this bit will initiate a SYNC event. |
|  | SYNC_PLL1_DLD | Engage SYNC mode until PLL1 DLD is true |
|  | SYNC_PLL2_DLD | Engage SYNC mode until PLL2 DLD is true |
|  | NO_SYNC_CLKoutX_Y | Synchronization will not affect selected clock outputs, where X = even-numbered output and Y = odd-numbered output. |
|  | SYNC_QUAL | Sets the SYNC to qualify mode for dynamic digital delay. |
|  | EN_SYNC | Must be set when using SYNC, but may be cleared after the SYNC event. When using dynamic digital delay (SYNC_QUAL = 1), EN_SYNC must always be set. Changing this value from 0 to 1 can cause a SYNC event, so clocks which should not be SYNCed when setting this bit should have the NO_SYNC_CLKoutX_Y bit set. NOTE: This bit is not a valid method of generating a SYNC event. Use one of the other SYNC generation methods to ensure a proper SYNC occurs. |
|  | SYNC_EN_AUTO | Enable auto SYNC when R0 to R5 is written. |
| DAC/Holdover | HOLDOVER_MODE | Sets holdover mode to be disabled or enabled. |
|  | FORCE_HOLDOVER | Engages holdover when checked regardless of HOLDOVER_MODE value. Turns the DAC on. |
|  | EN_TRACK | Enables DAC tracking. DAC tracks the PLL1 Vtune to provide for an accurate HOLDOVER mode. DAC_CLK_DIV should also be set so that DAC update rate is <= 100 kHz. |
|  | EN_VTUNE_RAIL_DET | Allows rail-to-rail operation of VCXO with default of 0. Allows use of DAC_LOW_TRIP, DAC_HIGH_TRIP. Must be used with EN_MAC_DAC = 1. CLKin_SELECT_MODE must be 4 or 6 (auto mode) to use. |
|  | HOLD_DLD_CNT | In HOLDOVER mode, wait for this many clocks of PLL1 PDF within the tolerances of PLL1_WND _SIZE before exiting holdover mode. |
|  | DAC_CLK_DIV | DAC update clock is the PLL1 phase detector divided by this divisor. For proper operation, DAC update clock rate should be <= 100 kHz. DAC update rate = PLL1 phase detector frequency / DAC_CLK_DIV |
|  | EN_MAN_DAC | Enables manual DAC mode and set DAC voltage when in holdover. |
|  | MAN_DAC | Sets the value for the DAC when EN_MAN_DAC is 1 and holdover is engaged. Readback from this register is the current DAC value whether in manual DAC mode or DAC tracking mode |
|  | DAC_LOW_TRIP | Value from GND in ~50mV steps at which a clock switch event is generated. If Holdover mode is enabled, it will be engaged upon the clock switch event. NOTE: EN_VTUNE_RAIL_DET must be enabled for this to be valid. |
|  | DAC_HIGH_TRIP | Value from VCC (3.3V) in ~50mV steps at which clock switch event is generated. If Holdover mode is enabled, it will be engaged upon the clock switch event. NOTE: EN_VTUNE_RAIL_DET must be enabled for this to be valid. |
| PLL1 | PLL1_WND_SIZE | If the phase error between the PLL1 reference and feedback clocks is less than specified time, then the PLL1 lock counter increments. NOTE: Final lock detect valid signal is determined when the PLL1 lock counter meets or exceeds the PLL1_DLD_CNT value. |
|  | PLL1_DLD_CNT | The reference and feedback of PLL1 must be within the window of phase error as specified by PLL1_WND_SIZE for this many cycles before PLL1 digital lock detect is asserted. |
|  | CLKinX_PreR_DIV | The PreR dividers divide the CLKinX reference before the PLL1_R divider. Unique divides on individual CLKinX signals allows switchover from one clock input to another clock input without needing to reprogram the PLL1_R divider to keep the device in lock. |
|  | PLL1_N_DLY | N delay causes clock outputs to lead clock input when in a 0-delay mode. Increasing the N delay value increases the output phase lead relative to the input. |
|  | PLL1_R_DLY | R delay causes clock outputs to lag clock input when in a 0-delay mode. Increasing the R delay value increases the output phase lag relative to the input. |
| PLL2 | PLL2_WND_SIZE | If the phase error between the PLL2 reference and feedback clock is less than specified time, then the PLL2 lock counter increments. |
|  | PLL2_DLD_CNT | The reference and feedback of PLL2 must be within the window of phase error as specified by PLL2_WND_SIZE for this many cycles before PLL2 digital lock detect is asserted. |
|  | EN_PLL2_REF_2X | Enables the doubler block to doubles the reference frequency into the PLL2 R counter. This can allow for frequency of 2/3, 2/5, etc. of OSCin to be used at the phase detector of PLL2. |
|  | PLL2_N_CAL | The PLL2_N_CAL register contains the N value used for the VCO calibration routine. Except during 0-delay modes, the PLL2_N and PLL2_N_CAL registers will be exactly the same. |
|  | PLL2_R3_LF | Set the corresponding integrated PLL2 loop filter values: R3, R4, C3, and C4. It is also possible to set these values by clicking on the loop filter values on the Clock Outputs tab. |
|  | PLL2_R4_LF |  |
|  | PLL2_C3_LF |  |
|  | PLL2_C4_LF |  |
|  | PLL2_FAST_PDF | Enable this bit when using a PLL2 phase detector frequency > 100 MHz. |
| Program Pins | SYNC | Sets these pins on the uWire header to logic high (checked) or logic low (unchecked). |
|  | Status_CLKin0 |  |
|  | Status_CLKin1 |  |

## Registers Tab

Figure 14: Registers Tab

The Registers tab shows the value of each register.  This is convenient for programming the device to the desired settings, then exporting to a text file the register values in hexadecimal for use in your own application.

By clicking in the “bit field” it is possible to manually change the value of registers by typing ‘1’ and ‘0.’

# Appendix B: Typical Phase Noise Performance Plots

## PLL1

The LMK048xxB’s dual PLL architecture achieves ultra low jitter and phase noise by allowing the external VCXO or Crystal’s phase noise to dominate the final output phase noise at low offset frequencies and the internal VCO’s phase noise to dominate the final output phase noise at high offset frequencies.  This results in the best overall noise and jitter performance.

Table 11 lists the test conditions used for output clock phase noise measurements with the Crystek 122.88 MHz VCXO.

Table 11:  LMK048xxB Test Conditions

| Parameter | Value |
| --- | --- |
| PLL1 Reference clock input | CLKin0 single-ended input, CLKin0* AC-coupled to GND |
| PLL1 Reference Clock frequency | 122.88 MHz |
| PLL1 Phase detector frequency | 122.88 MHz |
| PLL1 Charge Pump Gain | 100 uA |
| VCXO frequency | 122.88 MHz |
| PLL2 phase detector frequency | 122.88 MHz |
| PLL2 Charge Pump Gain | 3200 uA |
| PLL2 REF2X mode | Disabled |

### 122.88 MHz VCXO Phase Noise

The phase noise of the reference is masked by the phase noise of this VCXO by using a narrow loop bandwidth for PLL1 while retaining the frequency accuracy of the reference clock input.  This VCXO sets the reference noise to PLL2.  Figure 15 shows the open loop typical phase noise performance of the CVHD-950-122.88 Crystek VCXO.

Figure 15: Crystek CVHD-950-122.88 MHz VCXO Phase Noise at 122.88 MHz

| Table 12: VCXO Phase Noise at 122.88 MHz (dBc/Hz) Offset; Phase Noise. 10 Hz; -76.6. 100 Hz; -108.9. 1 kHz; -137.4. 10 kHz; -153.3. 100 kHz; -162.0. 1 MHz; -165.7. 10 MHz; -168.1. 40 MHz; -168.1. | Table 13: VCXO RMS Jitter to high offset of 20 MHz at 122.88 MHz (rms fs) Low Offset; Jitter. 10 Hz; 515.4. 100 Hz; 60.5. 1 kHz; 36.2. 10 kHz; 35.0. 100 kHz; 34.5. 1 MHz; 32.9. 10 MHz; 22.7. |
| --- | --- |

### Clock Output Measurement Technique

The same technique was used to measure phase noise for all three output types available on the programmable OSCout and CLKout buffers.  This was achieved by terminating one side of the LVPECL, LVDS, or LVCMOS output with a 50-ohm load, and measuring the other side single-ended using an Agilent E5052B Source Signal Analyzer.

### Buffered OSCout Phase Noise

Both OSCout0 and OSCout1 frequencies are 122.88 MHz since the OSCout Divider is bypassed.  OSCout0 is programmed to LVCMOS mode and OSCout1 is fixed as LVPECL.

## Clock Outputs (CLKout)

The LMK04800 Family features programmable LVDS, LVPECL, and LVCMOS buffer modes for the CLKoutX and OSCout0 output pairs.  The OSCout1 output pair has a LVPECL buffer.  Included below are various phase noise measurements for each output format.

### LMK04808B CLKout Phase Noise

Figure 16: LMK04808B CLKout Phase Noise

Table 14: LMK04808B Phase Noise (dBc/Hz) Phase Noise and RMS Jitter (fs)

| Offset | 1474.56 MHz LVDS | 1474.56 MHz LVPECL | 491.52 MHz LVDS | 491.52 MHz LVPECL |
| --- | --- | --- | --- | --- |
| 100 Hz | -88.2 | -87.8 | -97.7 | -99.7 |
| 1 kHz | -109.8 | -108.9 | -118.9 | -119.3 |
| 10 kHz | -118.0 | -117.4 | -127.3 | -127.4 |
| 100 kHz | -119.8 | -119.7 | -129.1 | -129.4 |
| 800 kHz | -130.9 | -130.8 | -140.1 | -140.3 |
| 1 MHz | -132.7 | -132.6 | -141.7 | -142.1 |
| 10 MHz | -148.2 | -149.2 | -152.7 | -154.9 |
| 20 MHz | -148.9 | -150.1 | -153.2 | -155.3 |
| RMS Jitter (fs) 10 kHz to 20 MHz | 99.3 | 99.4 | 109.1 | 103.3 |
| RMS Jitter (fs) 100 Hz to 20 MHz | 111.4 | 113.5 | 121.3 | 116.1 |

For the LMK04808B, the internal VCO frequency is 2949.12 MHz.  The divide-by-12 CLKout frequency is 245.76 MHz, and the divide-by-24 CLKout frequency is 122.88 MHz.

Table 15: LMK04808B Phase Noise and RMS Jitter for Different CLKout Output Formats and Frequencies

| Offset | 245.76 LVDS | 245.76 LVCMOS | 245.76 LVPECL | 122.88 LVDS | 122.88 LVCMOS | 122.88 LVPECL |
| --- | --- | --- | --- | --- | --- | --- |
| 100 Hz | -103.7 | -104.4 | -106.1 | -108.4 | -110.4 | -108.5 |
| 1 kHz | -123.7 | -125.9 | -125.7 | -128.7 | -129.7 | -130.9 |
| 10 kHz | -133.8 | -133.5 | -134.0 | -139.3 | -139.8 | -140.0 |
| 100 kHz | -135.5 | -135.4 | -135.8 | -141.7 | -141.4 | -141.8 |
| 800 kHz | -146.4 | -146.3 | -146.7 | -151.7 | -152.1 | -152.4 |
| 1 MHz | -147.9 | -148.0 | -148.3 | -152.9 | -153.4 | -153.8 |
| 10 MHz | -156.5 | -157.5 | -158.3 | -158.7 | -160.5 | -161.0 |
| 20 MHz | -156.9 | -157.8 | -158.6 | -158.8 | -160.7 | -161.3 |
| RMS Jitter (fs) 10 kHz to 20 MHz | 111.8 | 109.6 | 103.9 | 133.4 | 122.1 | 116.1 |
| RMS Jitter (fs) 100 Hz to 20 MHz | 124.3 | 122.1 | 115.4 | 144.4 | 132.3 | 127.9 |

### LMK04808B OSCout Phase Noise

Figure 17: LMK04808B OSCout Phase Noise

Table 16: LMK04808B OSCout Phase Noise and RMS Jitter (fs)

| Offset | OSCout0 LVPECL | OSCout1 LVPECL | OSCin thru CLKout |
| --- | --- | --- | --- |
| 100 Hz | -110.2 | -111.1 | -109.4 |
| 1 kHz | -137.2 | -136.1 | -138.6 |
| 10 kHz | -148.0 | -148.0 | -146.7 |
| 100 kHz | -157.8 | -156.0 | -155.3 |
| 800 kHz | -159.1 | -159.4 | -156.4 |
| 1 MHz | -157.0 | -157.5 | -156.3 |
| 10 MHz | -158.8 | -159.2 | -156.7 |
| 20 MHz | -159.2 | -159.5 | -156.7 |
| RMS Jitter (fs) 10 kHz to 20 MHz | 94.1 | 91.0 | 123.7 |
| RMS Jitter (fs) 100 Hz to 20 MHz | 103.2 | 99.6 | 131.3 |

### LMK04806B CLKout Phase Noise

Figure 18: LMK04806B CLKout Phase Noise

Table 17: LMK04806B Phase Noise (dBc/Hz) Phase Noise and RMS Jitter (fs)

| Offset | 1228.80 MHz LVDS | 1228.80 MHz LVPECL | 491.52 MHz LVDS | 491.52 MHz LVPECL |
| --- | --- | --- | --- | --- |
| 100 Hz | -91.2 | -90.5 | -97.5 | -98.2 |
| 1 kHz | -111.2 | -110.8 | -118.7 | -119.4 |
| 10 kHz | -121.0 | -121.1 | -128.5 | -128.6 |
| 100 kHz | -121.3 | -121.2 | -129.5 | -129.5 |
| 800 kHz | -133.8 | -133.7 | -141.9 | -141.9 |
| 1 MHz | -135.8 | -135.7 | -143.4 | -143.8 |
| 10 MHz | -150.2 | -150.4 | -153.1 | -155.7 |
| 20 MHz | -150.8 | -151.0 | -153.8 | -155.7 |
| RMS Jitter (fs) 10 kHz to 20 MHz | 92.9 | 93.4 | 97.5 | 94.5 |
| RMS Jitter (fs) 100 Hz to 20 MHz | 104.0 | 105.3 | 109.0 | 105.4 |

For the LMK04806B, the internal VCO frequency is 2457.60 MHz.  The divide-by-10 CLKout frequency is 245.76 MHz, and the divide-by-20 CLKout frequency is 122.88 MHz.

Table 18: LMK04806B Phase Noise and RMS Jitter for Different CLKout Output Formats and Frequencies

| Offset | 245.76 LVDS | 245.76 LVCMOS | 245.76 LVPECL | 122.88 LVDS | 122.88 LVCMOS | 122.88 LVPECL |
| --- | --- | --- | --- | --- | --- | --- |
| 100 Hz | -105.8 | -104.5 | -106.5 | -108.6 | -113.0 | -111.4 |
| 1 kHz | -124.7 | -124.9 | -125.4 | -130.2 | -132.1 | -131.0 |
| 10 kHz | -134.8 | -134.4 | -134.9 | -140.7 | -140.7 | -141.0 |
| 100 kHz | -135.5 | -135.4 | -135.8 | -141.7 | -141.7 | -141.9 |
| 800 kHz | -147.8 | -147.7 | -148.0 | -152.6 | -153.5 | -154.1 |
| 1 MHz | -149.6 | -149.4 | -149.7 | -153.3 | -155.2 | -155.5 |
| 10 MHz | -156.1 | -158.1 | -158.4 | -158.4 | -161.5 | -161.1 |
| 20 MHz | -156.3 | -158.2 | -158.9 | -159.5 | -161.6 | -161.3 |
| RMS Jitter (fs) 10 kHz to 20 MHz | 106.9 | 101.5 | 96.4 | 134.2 | 109.7 | 108.2 |
| RMS Jitter (fs) 100 Hz to 20 MHz | 116.8 | 112.4 | 106.4 | 143.2 | 118.9 | 117.9 |

### LMK04806B OSCout Phase Noise

Figure 19: LMK04806B OSCout Phase Noise

Table 19: LMK04806B OSCout Phase Noise and RMS Jitter (fs)

| Offset | OSCout0 LVPECL | OSCout1 LVPECL | OSCin thru CLKout |
| --- | --- | --- | --- |
| 100 Hz | -110.3 | -112.1 | -110.0 |
| 1 kHz | -136.9 | -137.9 | -138.9 |
| 10 kHz | -151.1 | -150.1 | -150.0 |
| 100 kHz | -154.3 | -156.8 | -154.6 |
| 800 kHz | -158.9 | -158.9 | -156.6 |
| 1 MHz | -159.2 | -159.1 | -156.6 |
| 10 MHz | -159.4 | -160.0 | -156.8 |
| 20 MHz | -157.6 | -159.9 | -156.9 |
| RMS Jitter (fs) 10 kHz to 20 MHz | 138.4 | 89.7 | 120.0 |
| RMS Jitter (fs) 100 Hz to 20 MHz | 143.7 | 97.3 | 126.2 |

### LMK04803B CLKout Phase Noise

Figure 20: LMK04803B CLKout Phase Noise

Table 20: LMK04803B Phase Noise (dBc/Hz) Phase Noise and RMS Jitter (fs)

| Offset | 983.04 MHz LVDS | 983.04 MHz LVPECL | 491.52 MHz LVDS | 491.52 MHz LVPECL |
| --- | --- | --- | --- | --- |
| 100 Hz | -93.0 | -95.0 | -99.7 | -99.5 |
| 1 kHz | -111.3 | -113.1 | -117.1 | -118.3 |
| 10 kHz | -121.8 | -121.1 | -126.9 | -127.4 |
| 100 kHz | -122.3 | -121.9 | -127.8 | -128.3 |
| 800 kHz | -134.9 | -134.5 | -140.3 | -140.8 |
| 1 MHz | -136.8 | -136.3 | -142.0 | -142.7 |
| 10 MHz | -150.3 | -150.6 | -152.0 | -154.2 |
| 20 MHz | -150.4 | -151.1 | -152.6 | -154.6 |
| RMS Jitter (fs) 10 kHz to 20 MHz | 103.8 | 107.4 | 116.0 | 108.1 |
| RMS Jitter (fs) 100 Hz to 20 MHz | 116.1 | 116.2 | 127.4 | 116.8 |

For the LMK04803B, the internal VCO frequency is 1966.08 MHz.  The divide-by-8 CLKout frequency is 245.76 MHz, and the divide-by-16 CLKout frequency is 122.88 MHz.

Table 21: LMK04803B Phase Noise and RMS Jitter for Different CLKout Output Formats and Frequencies

| Offset | 245.76 LVDS | 245.76 LVCMOS | 245.76 LVPECL | 122.88 LVDS | 122.88 LVCMOS | 122.88 LVPECL |
| --- | --- | --- | --- | --- | --- | --- |
| 100 Hz | -104.8 | -104.2 | -108.4 | -112.1 | -111.5 | -112.7 |
| 1 kHz | -124.8 | -123.5 | -125.0 | -130.5 | -130.2 | -130.6 |
| 10 kHz | -133.7 | -134.0 | -133.6 | -140.1 | -139.8 | -140.1 |
| 100 kHz | -134.4 | -134.4 | -134.6 | -140.6 | -140.4 | -140.7 |
| 800 kHz | -146.7 | -146.7 | -147.1 | -152.5 | -152.4 | -152.7 |
| 1 MHz | -148.5 | -148.5 | -148.7 | -153.4 | -154.0 | -154.0 |
| 10 MHz | -156.2 | -157.2 | -158.0 | -158.4 | -160.1 | -160.5 |
| 20 MHz | -156.6 | -157.5 | -158.2 | -158.7 | -160.4 | -161.1 |
| RMS Jitter (fs) 10 kHz to 20 MHz | 115.1 | 113.1 | 107.9 | 137.5 | 126.1 | 120.5 |
| RMS Jitter (fs) 100 Hz to 20 MHz | 126.4 | 124.2 | 116.7 | 147.5 | 136.4 | 128.9 |

### LMK04803B OSCout Phase Noise

Figure 21: LMK04803B OSCout Phase Noise

Table 22: LMK04803B OSCout Phase Noise and RMS Jitter (fs)

| Offset | OSCout0 LVPECL | OSCout1 LVPECL | OSCin thru CLKout |
| --- | --- | --- | --- |
| 100 Hz | -114.0 | -113.1 | -113.3 |
| 1 kHz | -136.6 | -137.2 | -138.4 |
| 10 kHz | -147.6 | -146.6 | -148.5 |
| 100 kHz | -156.3 | -156.2 | -154.0 |
| 800 kHz | -159.4 | -159.2 | -156.6 |
| 1 MHz | -159.0 | -159.3 | -156.7 |
| 10 MHz | -157.2 | -158.7 | -156.9 |
| 20 MHz | -158.1 | -159.4 | -157.0 |
| RMS Jitter (fs) 10 kHz to 20 MHz | 107.1 | 92.4 | 119.1 |
| RMS Jitter (fs) 100 Hz to 20 MHz | 111.6 | 98.1 | 123.3 |

# Appendix C: Schematics

## Power Supplies

## LMK048xxB Device with Loop Filter and Crystal Circuits

## Reference Inputs (CLKin0 & CLKin1), External VCXO (OSCin) & VCO Circuits, and OSCout1 Output

## Clock Outputs (OSCout0, CLKout0 to CLKout3)

## Clock Outputs (CLKout4 to CLKout7)

## Clock Outputs (CLKout8 to CLKout11)

## uWire Header, Logic I/O Ports and Status LEDs

## USB Interface

# Appendix D: Bill of Materials

Table 23: Bill of Materials for LMK048xx Evaluation Boards

| Item | Designator | Description | Manufacturer | PartNumber | Quantity |
| --- | --- | --- | --- | --- | --- |
| 1 | C1, C5, C13, C20, C22, C25, C300, R3, R11, R12, R19, R21, R22, R29, R30, R37, R46, R55, R73, R74, R82, R84, R229, R304, R327, R329, R331, R333, R337, R340, R341, R346, R347, R349, R353, R354, R358, R361, R364, R365, R368, R371, R373, R374, R375, R400, R402 | FB, 120 ohm, 500 mA, 0603, RES, 0 ohm, 5%, 0.1W, 0603 | Vishay-Dale, Vishey/Dale | CRCW06030000Z0EA | 47 |
| 2 | C1_A1, C6, C14, C19, C21, C24, C27, C37, C38, C44, C46, C47, C48, C51, C52, C53, C54, C57, C58, C59, C60, C63, C64, C65, C66, C70, C71, C72, C75, C76, C77, C78, C81, C82, C312, C319, C329, C361 | CAP, CERM, 0.1uF, 25V, +/-5%, X7R, 0603 | Kemet | C0603C104J3RACTU | 38 |
| 3 | C1_A2 | CAP, CERM, 47pF, 50V, +/-5%, C0G/NP0, 0603 | Kemet | C0603C470J5GACTU | 1 |
| 4 | C1_B2 | CAP, CERM, 150pF, 50V, +/-5%, C0G/NP0, 0603 | Kemet | C0603C151J5GACTU | 1 |
| 5 | C2, C12, C41, C302, C330, C346 | CAP, CERM, 0.1uF, 16V, +/-10%, X7R, 0603 | Kemet | C0603C104K4RACTU | 6 |
| 6 | C2pB2 | CAP, CERM, 0.12uF, 50V, +/-10%, X7R, 0805 | Kemet | C0805C124K5RACTU | 1 |
| 7 | C2_A1 | CAP, CERM, 0.68µF, 10V, +/-10%, X5R, 0603 | Kemet | C0603C684K8PAC | 1 |
| 8 | C2_A2 | CAP, CERM, 3900pF, 50V, +/-10%, X7R, 0603 | MuRata | GRM188R71H392KA01D | 1 |
| 9 | C4, C69, C314, C322, C326, C367, C400, C401, C402, C409, C412, C414 | CAP, CERM, 0.1uF, 25V, +/-10%, X7R, 0603, CAP, CERM, 0.1µF, 25V, +/-10%, X7R, 0603 | Kemet | C0603C104K3RACTU | 12 |
| 10 | C9 | CAP, CERM, 82pF, 50V, +/-10%, C0G/NP0, 0603 | Kemet | C0603C820K5GACTU | 1 |
| 11 | C10, C29, C32, C341 | CAP, CERM, 2200pF, 50V, +/-10%, X7R, 0603 | Kemet | C0603C222K5RACTU | 4 |
| 12 | C11 | CAP, CERM, 10µF, 10V, +/-20%, X5R, 0805 | Kemet | C0805C106M8PACTU | 1 |
| 13 | C23, C26, R103, R104, R111, R112, R125, R126, R133, R134, R213, R215, R221, R222, R231, R232, R338 | RES, 240 ohm, 5%, 0.1W, 0603 | Vishay-Dale | CRCW0603240RJNEA | 17 |
| 14 | C28, C34 | CAP, CERM, 2pF, 50V, +/-12.5%, C0G/NP0, 0603 | Kemet | C0603C209C5GACTU | 2 |
| 15 | C35, C310, C317, C324, C352, Cb2pB1 | CAP, CERM, 10uF, 10V, +/-10%, X5R, 0805 | Kemet | C0805C106K8PACTU | 6 |
| 16 | C304 | CAP, CERM, 1000pF, 50V, +/-5%, C0G/NP0, 0603 | Kemet | C0603C102J5GACTU | 1 |
| 17 | C311, C313, C318, C321, C325, C331, C342 | CAP, CERM, 1uF, 10V, +/-10%, X5R, 0603 | Kemet | C0603C105K8PACTU | 7 |
| 18 | C315, C323 | CAP, CERM, 0.01uF, 100V, +/-10%, X7R, 0603 | Kemet | C0603C103K1RACTU | 2 |
| 19 | C340 | CAP, CERM, 4.7uF, 10V, +/-10%, X5R, 0603 | Kemet | C0603C475K8PACTU | 1 |
| 20 | C350, C351, C359, C360 | CAP, CERM, 0.47uF, 16V, +/-10%, X7R, 0603 | Kemet | C0603C474K4RACTU | 4 |
| 21 | C368 | CAP, CERM, 100pF, 50V, +/-5%, C0G/NP0, 0603 | Kemet | C0603C101J5GACTU | 1 |
| 22 | C403 | CAP, CERM, 10uF, 6.3V, +/-10%, X5R, 0805 | Kemet | C0805C106K9PAC | 1 |
| 23 | C404 | CAP, TANT, 1uF, 20V, +/-10%, 8.4 ohm, 3216-18 SMD | Vishay-Sprague | 293D105X9020A2TE3 | 1 |
| 24 | C405, C408 | CAP, CERM, 0.22uF, 16V, +/-10%, X7R, 0603 | Kemet | C0603C224K4RACTU | 2 |
| 25 | C406, C407 | CAP, CERM, 18pF, 50V, +/-5%, C0G/NP0, 0603 | Kemet | C0603C180J5GACTU | 2 |
| 26 | C410, C411 | CAP, CERM, 1uF, 16V, +/-10%, X5R, 0603 | Kemet | C0603C105K4PACTU | 2 |
| 27 | C413, C415 | CAP, CERM, 0.01uF, 50V, +/-5%, X7R, 0603 | Kemet | C0603C103J5RACTU | 2 |
| 28 | Cb1_B1 | CAP, CERM, 0.33uF, 16V, +/-10%, X7R, 0603 | Kemet | C0603C334K4RACTU | 1 |
| 29 | CLKin0, CLKin0*, CLKout0, CLKout0*, CLKout2, CLKout2*, CLKout4, CLKout4*, CLKout6, CLKout6*, CLKout8, CLKout8*, CLKout10, CLKout10*, FBCLKin*/CLKin1*, OSCout0, OSCout0*, OSCout1, OSCout1* | Connector, SMT, End launch SMA 50 Ohm | Emerson Network Power | 142-0701-851 | 19 |
| 30 | CR400, CR401 | ESD suppressor | Littelfuse Inc | PGB1010603MR | 2 |
| 31 | D1 | Common Cathode Tuning Varactor | Skyworks | SMV1249-074LF | 1 |
| 32 | D2, D3, D4 | LED 2.8X3.2MM 565NM RED CLR SMD | Lumex Opto/Components Inc. | SML-LX2832IC | 3 |
| 33 | D5 | LED 2.8X3.2MM 565NM GRN CLR SMD | Lumex Opto/Components Inc. | SML-LX2832GC | 1 |
| 34 | J1 | CONN TERM BLK PCB 5.08MM 2POS OR | Weidmuller | 1594540000 | 1 |
| 35 | R2, R13, R36, R47, R59, R70, R332 | RES, 0 ohm, 5%, 0.125W, 0805 | Vishay-Dale | CRCW08050000Z0EA | 7 |
| 36 | R2_A1 | RES, 39k ohm, 5%, 0.1W, 0603 | Vishay-Dale | CRCW060339K0JNEA | 1 |
| 37 | R2_A2 | RES, 620 ohm, 5%, 0.1W, 0603 | Vishay-Dale | CRCW0603620RJNEA | 1 |
| 38 | R2_B2 | RES, 470 ohm, 5%, 0.1W, 0603 | Vishay-Dale | CRCW0603470RJNEA | 1 |
| 39 | R4, R9 | RES, 100 ohm, 5%, 0.1W, 0603 | Vishay-Dale | CRCW0603100RJNEA | 2 |
| 40 | R18, R336, R342, R343 | FB, 1000 ohm, 600 mA, 0603, Ferrite | Murata | BLM18HE102SN1D | 4 |
| 41 | R24 | RES, 18 ohm, 5%, 0.1W, 0603 | Vishay-Dale | CRCW060318R0JNEA | 1 |
| 42 | R26, R27, R83, R85 | RES, 270 ohm, 5%, 0.1W, 0603 | Vishay-Dale | CRCW0603270RJNEA | 4 |
| 43 | R43, R107, R108, R129, R130, R151, R152, R173, R174, R195, R196, R217, R218 | RES, 68 ohm, 5%, 0.1W, 0603 | Vishay-Dale | CRCW060368R0JNEA | 13 |
| 44 | R61, R102, R117, R124, R138, R142, R158, R164, R182, R190, R206, R212, R228 | RES, 51 ohm, 5%, 0.1W, 0603 | Vishay-Dale | CRCW060351R0JNEA | 13 |
| 45 | R62, R67 | RES, 4.7k ohm, 5%, 0.1W, 0603 | Vishay-Dale | CRCW06034K70JNEA | 2 |
| 46 | R80, R81, R312, R315, R317, R318, R321, R324 | RES, 15k ohm, 5%, 0.1W, 0603 | Vishay-Dale | CRCW060315K0JNEA | 8 |
| 47 | R233, R234 | RES, 33 ohm, 5%, 0.1W, 0603 | Vishay-Dale | CRCW060333R0JNEA | 2 |
| 48 | R307 | RES, 10k ohm, 5%, 0.1W, 0603 | Vishay-Dale | CRCW060310K0JNEA | 1 |
| 49 | R313, R316, R319, R320, R322, R325 | RES, 27k ohm, 5%, 0.1W, 0603 | Vishay-Dale | CRCW060327K0JNEA | 6 |
| 50 | R344 | RES, 392 ohm, 1%, 0.1W, 0603 | Vishay-Dale | CRCW0603392RFKEA | 1 |
| 51 | R345, R348, R355, R359, R362, R366, R372 | FB, 120 ohm, 500 mA, 0603 | Murata | BLM18AG121SN1D | 7 |
| 52 | R350, R369 | RES, 51k ohm, 5%, 0.1W, 0603 | Vishay-Dale | CRCW060351K0JNEA | 2 |
| 53 | R351 | RES, 2.00k ohm, 1%, 0.1W, 0603 | Vishay-Dale | CRCW06032K00FKEA | 1 |
| 54 | R356 | RES, 866 ohm, 1%, 0.1W, 0603 | Vishay-Dale | CRCW0603866RFKEA | 1 |
| 55 | R404, R405 | RES, 22 ohm, 5%, 0.125W, 0805 | Vishay-Dale | CRCW080522R0JNEA | 2 |
| 56 | R407, R411 | RES, 47k ohm, 5%, 0.1W, 0603 | Vishay-Dale | CRCW060347K0JNEA | 2 |
| 57 | Rb2_B1 | RES, 3.9k ohm, 5%, 0.1W, 0603 | Vishay-Dale | CRCW06033K90JNEA | 1 |
| 58 | S1, S2, S3, S4, S5, S6 | 0.875" Standoff | VOLTREX | SPCS-14 | 6 |
| 59 | U1 | LMK04800 | Texas Instruments | LMK048xxBISQ | 1 |
| 60 | U2 | 122.88 MHz VCXO | Crystek | CVHD-950-122.88 | 1 |
| 61 | U4, U300 | Precison Single Low Noise, Low 1/F corner Op Amp | Texas Instruments | LMP7731MF | 2 |
| 62 | U302 | Micropower 800mA Low Noise 'Ceramic Stable' Adjustable Voltage Regulator for 1V to 5V Applications | Texas Instruments | LP3878SD-ADJ | 1 |
| 63 | U303, U305 | Ultra Low Noise, 150mA Linear Regulator for RF/Analog Circuits Requires No Bypass Capacitor | Texas Instruments | LP5900SD-3.3 | 2 |
| 64 | uWire | Low Profile Vertical Header 2x5 0.100" | FCI | 52601-G10-8LF | 1 |
| 65 | Vcc | Connector, TH, SMA | Emerson Network Power | 142-0701-201 | 1 |
| 66 | VccVCO/Aux, VccVCXO/Aux | Connector, SMA Jack, Vertical, Gold, SMD | Emerson Network Power Connectivity | 142-0711-201 | 2 |
| Unpopulated Components |  |  |  |  |  |
| 67 | B1, B2, B3 | ADT2-1T Balun | MiniCircuits | ADT2-1T+ | 0 |
| 68 | C2pA1 | CAP, CERM, 2.7uF, 10V, +/-10%, X5R, 0805 | Kemet | C0805C275K8PACTU | 0 |
| 69 | C2pA2, C3_AB1, C15, C17, C42, C43, C306, C307, C308, C309 | CAP, CERM, 100pF, 50V, +/-5%, C0G/NP0, 0603 | Kemet | C0603C101J5GACTU | 0 |
| 70 | C2_B2 | CAP, CERM, 6800pF, 100V, +/-10%, X7R, 0603 | Kemet | C0603C682K1RACTU | 0 |
| 71 | C3, C36, C335 | CAP, CERM, 0.1uF, 25V, +/-5%, X7R, 0603 | Kemet | C0603C104J3RACTU | 0 |
| 72 | C7, C8, C16, C18, C33, C45, C49, C50, C55, C56, C61, C62, C67, C68, C73, C74, C79, C80, C301, C303, C305 | CAP, CERM, 0.1uF, 16V, +/-10%, X7R, 0603 | Kemet | C0603C104K4RACTU | 0 |
| 73 | C39, C40, C333, Cb1_VCO, Cb2_B1, Cb2_VCO, Cb2pVCO | Technology, Dielectric, xxV, xx% |  | DNP_CAP | 0 |
| 74 | C316, C320, C327, C328, C332, C334, C336 | CAP, CERM, 0.1uF, 16V, +/-5%, X7R, 0603 | Kemet | C0603C104J4RACTU | 0 |
| 75 | C337, C343, C347, C353, C356, C364 | CAP, CERM, 1uF, 10V, +/-10%, X5R, 0603 | Kemet | C0603C105K8PACTU | 0 |
| 76 | C338, C344, C348, C354, C357, C365 | CAP, CERM, 0.1uF, 25V, +/-10%, X7R, 0603 | Kemet | C0603C104K3RACTU | 0 |
| 77 | C339, C345, C349, C355, C358, C366 | CAP, CERM, 0.01uF, 100V, +/-10%, X7R, 0603 | Kemet | C0603C103K1RACTU | 0 |
| 78 | CLKin0_SEL, CLKin1_SEL, Status_Hold, Status_LD, SYNC | Connector, SMA Jack, Vertical, Gold, SMD | Emerson Network Power Connectivity | 142-0711-201 | 0 |
| 79 | CLKout1, CLKout1*, CLKout3, CLKout3*, CLKout5, CLKout5*, CLKout7, CLKout7*, CLKout9, CLKout9*, CLKout11, CLKout11*, FBCLKin/CLKin1, OSCin, OSCin*, Vtune1 | Connector, SMT, End launch SMA 50 Ohm | Emerson Network Power | 142-0701-851 | 0 |
| 80 | D300 | Diode, Zener, 3.3V, 150mW, SOD-523F | Comchip Technology | CZRU52C3V3 | 0 |
| 81 | GND_TP, Vcc_TP |  |  | DNP_TP | 0 |
| 82 | HWB, RESET |  | APEM Components, LLC | ADTSM31NV | 0 |
| 83 | J400 | CONN RCPT MINI USB2.0 5POS SMD | Hirose Electric Co. Ltd. | UX60SA-MB-5ST | 0 |
| 84 | R1, R20, R23, R51, R52, R53, R54, R60, R63, R66, R69, R71, R72, R75, R76, R77, R78, R230, R300, R301, R305, R306, R308, R335, R339, R352, R363, R367, R370 | RES, 0 ohm, 5%, 0.1W, 0603 | Vishay-Dale | CRCW06030000Z0EA | 0 |
| 85 | R5, R8, R14, R25, R33, R34, R41, R48, R64, R68, R90, R96, R99, R114, R121, R135, R141, R157, R163, R179, R187, R204, R209, R225, R376 | RES, 51 ohm, 5%, 0.1W, 0603 | Vishay-Dale | CRCW060351R0JNEA | 0 |
| 86 | R6, R7, R15, R17, R31, R32, R39, R40, R49, R50, R79, R314 | RES, 270 ohm, 5%, 0.1W, 0603 | Vishay-Dale | CRCW0603270RJNEA | 0 |
| 87 | R10, R16, R88, R94, R97, R100, R113, R115, R119, R122, R136, R139, R143, R145, R159, R161, R165, R167, R180, R183, R185, R188, R201, R202, R207, R210, R223, R226 | RES, 120 ohm, 5%, 0.1W, 0603 | Vishay-Dale | CRCW0603120RJNEA | 0 |
| 88 | R28, R42, R65 | RES, 100 ohm, 5%, 0.1W, 0603 | Vishay-Dale | CRCW0603100RJNEA | 0 |
| 89 | R35, R44, R89, R95, R98, R101, R116, R118, R120, R123, R137, R140, R144, R146, R160, R162, R166, R168, R181, R184, R186, R189, R203, R205, R208, R211, R224, R227 | RES, 82 ohm, 5%, 0.1W, 0603 | Vishay-Dale | CRCW060382R0JNEA | 0 |
| 90 | R38, R45, R91, R93, R105, R106, R109, R110, R127, R128, R131, R132, R149, R150, R153, R154, R171, R172, R175, R176, R193, R194, R197, R198, R214, R216, R219, R220 | RES, 62 ohm, 5%, 0.1W, 0603 | Vishay-Dale | CRCW060362R0JNEA | 0 |
| 91 | R86, R87 | RES, 27k ohm, 5%, 0.1W, 0603 | Vishay-Dale | CRCW060327K0JNEA | 0 |
| 92 | R92 | RES, 68 ohm, 5%, 0.1W, 0603 | Vishay-Dale | CRCW060368R0JNEA | 0 |
| 93 | R147, R148, R155, R156, R169, R170, R177, R178, R191, R192, R199, R200 | RES, 240 ohm, 5%, 0.1W, 0603 | Vishay-Dale | CRCW0603240RJNEA | 0 |
| 94 | R302, R303, R310, R311, R401 | RES, 10k ohm, 5%, 0.1W, 0603 | Vishay-Dale | CRCW060310K0JNEA | 0 |
| 95 | R309, Rb2_VCO | x%, x.xxxW |  | DNP_RES | 0 |
| 96 | R323 | RES, 2.2k ohm, 5%, 0.1W, 0603 | Vishay-Dale | CRCW06032K20JNEA | 0 |
| 97 | R326, R328, R330 | FB, 1000 ohm, 600 mA, 0603 | Murata | BLM18HE102SN1D | 0 |
| 98 | R334 | RES, 0 ohm, 5%, 0.125W, 0805 | Vishay-Dale | CRCW08050000Z0EA | 0 |
| 99 | R360 | RES, 51k ohm, 5%, 0.1W, 0603 | Vishay-Dale | CRCW060351K0JNEA | 0 |
| 100 | R403, R406, R408, R409, R410 | RES, 33 ohm, 5%, 0.1W, 0603 | Vishay-Dale | CRCW060333R0JNEA | 0 |
| 101 | U3 | VCO |  | CRO2949A-LF | 0 |
| 102 | U301 | 3-Terminal Adjustable Regulator | Texas Instruments | LM317AEMP | 0 |
| 103 | U400, Y400 |  |  |  | 0 |
| 104 | Y300, Y301 |  |  | DNP_XTAL | 0 |

# Appendix E:  PCB Layers Stackup

RO4003 (Er = 3.3)

16 mil

Top Layer [LMK048xxENG.GTL]

RF Ground plane [LMK048xxENG.G1]

FR4 (Er = 4.8)

4 mil

Power plane #1 [LMK048xxENG.G2]

FR4

12.6 mil

Ground plane [LMK048xxENG.GP1]

FR4

12 mil

Bottom Layer [LMK048xxENG.GBL]

62.2 mil thick

FR4

8 mil

Power plane #2 [LMK048xxENG.G3]

6-layer PCB Stackup includes:

- Top Layer for high-priority high-frequency signals (2 oz.)

- RO4003 Dielectric, 16 mils

- RF Ground plane (1 oz.)

- FR4, 4 mils

- Power plane #1 (1 oz.)

- FR4, 12.6 mils

- Ground plane (1 oz.)

- FR4, 8 mils

- Power Plane #2 (1 oz.)

- FR4, 12 mils

- Bottom Layer copper clad for thermal relief (2 oz.)

# Appendix F: PCB Layout

## Layer #1 – Top

## Layer #2 – RF Ground Plane (Inverted)

## Layer #3 – Vcc Planes

## Layer #4 – Ground Plane (Inverted)

## Layer # 5 – Vcc Planes 2

## Layer #6 – Bottom

## Layers #1 and 6 – Top and Bottom (Composite)

# Appendix G: EVM Software and Communication: Interfacing uWire

Codeloader is the software used to communicate with the EVM (Please download the latest version from TI.com - http://www.ti.com/tool/codeloader). This EVM can be controlled through the uWire interface on board. There are two options in communicating with the uWire interface from the computer.

OPTION 1

Open Codeloader.exe  Click “Select Device”  Click “Port Setup” tab  Click “LPT” (in Communication Mode)

OPTION 2

The Adapter Board

This table describes the pins configuration on the adapter board for each EVM board (See examples below table)

| EVM | Jumper Bank |  |  |  |  |  |  |  | Code Loader Configuration |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A | B | C | D | E | F | G | H |  |
| LMX2581 | A4 | B1 | C2 |  | E5 | F1 | G1 | H1 | BUFEN (pin 1), Trigger (pin 7) |
| LMX2541 | A4 |  | C3 |  | E4 | F1 | G1 | H1 | CE (pin 1), Trigger (pin 10) |
| LMK0400x | A0 |  | C3 |  | E5 | F1 | G1 | H1 | GOE (pin 7) |
| LMK01000 | A0 |  | C1 |  | E5 | F1 | G1 | H1 | GOE (pin 7) |
| LMK030xx | A0 |  | C1 |  | E5 | F1 | G1 | H1 | SYNC (pin 7) |
| LMK02000 | A0 |  | C1 |  | E5 | F1 | G1 | H1 | SYNC (pin 7) |
| LMK0480x | A0 | B2 | C3 |  | E5 | F0 | G0 | H1 | Status_CLKin1 (pin 3) |
| LMK04816/4906 | A0 | B2 | C3 |  | E5 | F0 | G0 | H1 | Status_CLKin1 (pin 3) |
| LMK01801 | A0 | B4 | C5 |  | E2 | F0 | G0 | H1 | Test (pin 3), SYNC0 (pin 10) |
| LMK0482x (prelease) | A0 | B5 | C3 | D2 | E4 | F0 | G0 | H1 | CLKin1_SEL (pin 6), Reset (pin 10) |
| LMX2531 | A0 |  |  |  | E5 | F2 | G1 | H2 | Trigger (pin 1) |
| LMX2485/7 | A0 |  | C1 |  | E5 | F2 | G1 | H0 | ENOSC (pin 7), CE (pin 10) |
| LMK03200 | A0 |  |  |  | E5 | F0 | G0 | H1 | SYNC (pin 7) |
| LMK03806 | A0 |  | C1 |  | E5 | F0 | G0 | H1 |  |
| LMK04100 | A0 |  | C1 |  | E5 | F1 | G1 | H1 |  |

Example adapter configuration (LMK01801)

Open Codeloader.exe  Click “Select Device”  Click “Port Setup” Tab  Click “USB” (in Communication Mode)

*Remember to also make modifications in “Pin Configuration” Section according to Table above.

# Appendix H: Troubleshooting Information

If the evaluation board is not behaving as expected, the most likely issues are…

- Board communication issue

- Incorrect Programming of the device

- Setup Error

Refer to this checklist for a practical guide on identifying/exposing possible issues.

## Confirm Communications

## Confirm PLL1 operation/locking

  - Program LD_MUX = “PLL1_R/2”

  - Confirm that LD pin output is half the expected phase detector frequency of PLL1.

    - If not, examine CLKin_SEL programming.

    - If not, examine CLKin0_BUFTYPE / CLKin1_BUFTYPE.

    - If not, examine PLL1 register R programming.

    - If not, examine physical CLKin input.

  - Program LD_MUX = “PLL1_N /2”

  - Confirm that LD pin output is half the expected phase detector frequency of PLL1.

    - If not, examine PLL1 register N programming.

    - If not, examine physical OSCin input.

Naturally, the output frequency of the above two items, PLL 1 R Divider/2 and PLL 1 N Divider /2, on LD pin should be the same frequency.

  - Program LD_MUX = “PLL1_DLD”

  - Confirm the LD pin output is high.

  - If high, then PLL1 is locked, continue to PLL2 operation/locking.

  - If LD pin output is low, but the frequencies are the same, it is possible that excessive leakage on Vtune pin is causing the digital lock detect to not activate.  By default PLL2 waits for the digital lock detect to go high before allowing PLL2 and the integrated VCO to lock.  Different VCXO models have different input leakage specifications.  High leakage, low PLL1 phase detector frequencies, and low PLL1 charge pump current settings can cause the PLL1 charge pump to operate longer than the digital lock detect timeout which allows the device to lock, but prevents the digital lock detect from activating.

- Redesign PLL1 loop filter with higher phase detector frequency

- Redesign PLL1 loop filter with higher charge pump current

- Isolate VCXO tuning input from PLL1 charge pump with an op amp.

## Confirm PLL2 operation/locking

  - Program LD_MUX = “PLL2_R/2”

  - Confirm that LD pin output is half the expected phase detector frequency of PLL2.

    - If not, examine PLL2_R programming.

    - If not, examine physical OSCin input.

  - Program LD_MUX = “PLL2_N/2”

  - Confirm that LD pin output is half the expected phase detector frequency of PLL2.

    - If not, confirm OSCin_FREQ is programmed to OSCin frequency.

    - If not, examine PLL2_N programming.

Naturally, the output frequency of the above two items should be the same frequency.

  - Program LD_MUX = “PLL2 DLD”

  - Confirm the LD pin output is high.

  - Program LD_MUX = “PLL1 & PLL2 DLD”

  - Confirm the LD pin output is high.